# Final Report

25th of March 2015

2IO70

Version 1

This document will contain the documents of the preceding phases and give an introduction and conclusion to the project. "The Final Report presents the reader with a clear picture of the designed machine, the method of working followed, the specification, validation, and design of the software, and a motivation of the main design decisions." (Source: Project Guide Design Based Learning "DBL 2IO70" "Sort It Out")

Group 16

Rolf Verschuuren

Wigger Boelens

Stefan van den Berg

Dat Phung

Maarten Keet

Tudor Petrescu

# Introduction

In this document you will find the details on how we have designed and built, in the past eight weeks, a sorting machine and the software that runs it. This Final Report will contain the five “Product” documents previously handed in and approved by the tutor, and a “Process” document. The five Product documents explain how we arrived at our final design for both the hardware and the software. In order of when we made them, these are “Machine Design”, where we explain how the machine was designed and why we chose to do it that way. Subsequently comes “Software Specification”, where we made a finite state automaton that the software was going to be based on. Then came the “Software Design” and “Software Implementation and Integration” documents in which we first designed the full program in pseudo-Java code and then subsequently translated this into working Assembly code. Throughout this document there are validation segments in which we explain how we validated our decisions. In the “Validation and Testing” document we look back at these segments and describe the measures we took to ensure that our final product would meet the initial requirements. The second part of the Final Report is the Process document, in this document we describe how we worked as a group over the course of this project, and how we decided to tackle any issues that arose. This Final Report is the final deliverable for the course.

Table of Contents

Introduction	2

Product	6

Machine Design	6

High level Specification	6

The specification as given in the Technical Guide	6

Our specification	6

Priorities	7

USE-cases	8

Starting the machine	8

Stop the machine	8

Sort unsorted disks	9

Abort the process	9

Booting of the machine	10

Shutting down the machine	10

User Constraints	11

Safety Properties	11

Explanation of Safety Properties	11

Design Decisions	12

The Feeder	12

The Transportation and Scanning	13

The sorting mechanism	15

Machine interface	16

The feeder	16

The position sensor	16

The black white detector	16

The Sorter	17

The buttons	17

The conveyer belt	17

System Validation and Testing	18

Validate High level specifications	18

Validation System Level Requirements	18

Validation Priorities to SLRs	19

Testing machine design to the priorities	20

Software Specification	21

Inputs and Outputs	21

Inputs	21

Outputs	23

Validation of “Inputs and Outputs”	23

Relations	24

Lens lamp of the black white detector	24

Lens lamp of the position sensor	24

Engine of the conveyor belt	24

The engine of on the conveyor belt only reacts to the input of the “START/STOP” button and the “ABORT” button.  The engine will start when the machine is in its resting state and the “START/STOP” button is pressed. If however the “START/STOP” button is pressed and the machine is not in its resting state then the machine will stop after it completed its current cycle.  Whenever the “ABORT” button is pressed the engine stops within 50ms.	24

Engine of the feeder	24

The engine for the feeder also only reacts to the input of the “START/STOP” button and the “ABORT” button. This engine  also starts when the machine is in its resting state and the “START/STOP” button is pressed. If however the machine is running then the engine will stop. When the “ABORT” button is pressed the engine stops within 50ms.	24

Engine for the sorter	24

Display for counting	24

Validation of “Relations”	25

Design Decisions	26

Description of States	27

Validation of “Description of States”	33

Starting the machine	33

Stopping the machine	34

Sort unsorted discs	35

Abort the process	36

State transitions	37

Validation of “State Transitions”	38

Finite-state Automaton	39

Validation of “Finite-state Automaton”	39

UPPAAL model	40

Tests done	40

Validation of “UPPAAL model”	40

Software Design	41

Translating from Java to PHP	41

Design decisions for the Java code	42

Validation of java to transition table	43

Validation of timerManage	43

Control flow validation	43

Software Implementation and Integration	44

Java to PHP	44

Validation of Java to PHP	44

Validation of PHP compiler to Assembly	44

Preprocessing	44

Splitting	44

Compiling	45

Combining	45

Formatting	45

System Validation and Testing	46

Process	47

Conclusion	48

Appendix 2: Java Program	50

Appendix 3: Explanation of the compiler	53

Appendix 4: Explanation of the compiler functions	55

Appendix 5: PHP Program	59

Appendix 6: Assembly code	63

# Product

# Machine Design

In this phase, we explain the design of our machine and how we decided on this design, and why we decided on this design. To do this we will take a look at our requirements and priorities. Afterwards we will look at the design and the decisions leading to that design.

# High level Specification

#### The specification as given in the Technical Guide

The goal of this project is to build a simple sorting machine that is able to separate small objects, plastic discs that may be either black or white, into two sets: the black discs and the white discs. (…) The machine must contain at least one conveyor belt.

(…)

The machine is to be operated by means of two push buttons, called “START/STOP” and “ABORT” (…) By pressing button “START/STOP” the machine is started. (…) If 4 seconds after (…) expected arrival time the presence detector has not signalled the arrival of a disc (…) the machine stops (…) If, during the sorting process. The push button “START/STOP” is pressed the machine (…) continues its normal operation until the current disc has been deposited into the correct tray. Then, the machine stops. (…) Push button “ABORT” (…) makes the machine halt immediately. (…) Pressing this button while the machine is in its resting state has no effect. (…) If subsequently, the push button “START/STOP” is pressed once, the machine returns to its resting state.

To be able to guarantee that the mechanism depositing discs onto the conveyor belt stops in a well-defined state, this mechanism must be equipped with (at least) one switch to signal that this mechanism has reached the correct state.

## Our specification

We have to make a so-called sorting machine. This machine should be able to separate, by colour, small black and white plastic discs. The requirements are as follows, the machine should:

- Have at least one conveyor belt.

- Have two buttons called “START/STOP” and “ABORT”.

- Start when the machine is in a resting state and “START/STOP” is pressed.

- Stop when the machine is running and “START/STOP” is pressed, before stopping it should sort all discs that are on the belt.

- Abort when “ABORT” is pressed, this should halt the machine immediately unless it’s in the resting state.

- Go to a resting state when the machine is in a halting state and “START/STOP” is pressed.

- Have at least one switch to signal when the machine is in a resting state.

# Priorities

1.	We define reliability as the ability of the machine to correctly sort all the inputted disks. We validate the reliability of the machine by checking the correctness of the code running the machine and also by conducting long-term test. Reliability is mainly reflected in our decision to encase the conveyer belt so that it is prevented any possibility of the discs, that are transported, to slip out. The goal of the project cannot be met with an unreliable design.

2.	The speed of the machine is defined by the number of disks sorted in a unit of time. We search to select the design solution that improves this number. Speed is essential to offer a pleasant experience operating the machine. Speed is also the first thing that stands out when two machines of this sort are compared.

3.	We define robustness as the fact that the machine does not break easily. The validation is if the machines state wouldn’t be changed, they wouldn’t break during: build phase, test phases, simulations, transportation and the end process, all during the period of the project cycle. Then we can consider the machine to be robust. Robustness can be observed from our design solution from the partial encasing used. Also the disc container was design to be robust do to its shape, size and simplicity. We do not meet our project goal if the machine isn’t capable of running during the final process.

4.	We define user accessibility as the ease in which the user takes the actions required from the machine. Validation is done by checking the compatibility of the design and the user constrains. The disc container was built with user accessibility in mind, it is fairly easy and fast to load discs. The reason why this priority is important is that the machine requires a user to be operated and in consequence its operation must be possible.

5.	We define amount of space by the amount of floor space that the machine occupies. Checking if there are useless components in the machine or other components that can be replaced with smaller counterparts without influencing the priorities above does validation of the low amount of space. From this perspective the current Feeder occupies a small amount a space, while the other feeder design would of forced us to add an extra floor extension because of its large dimensions. The reason of this priority is to ease the transportation and storage of the machine.

6.	The Difficulty of Building is self-explanatory. We validate this be checking if there are any useless components. In our decision to have the conveyer belt larger, trying to fit on the platform size, we simplified the design and left more physical space to work on the other components connected to the machine. Opting for such a priority would make our solution easy to implement.

7.	The Amount of Parts of the Machine is also self-explanatory. We also check if there are any useless parts. An example were we used very little parts by choice in our machine is the feeder component. Reasons why we picked this priority is that it might improve the overview of the machine and also the error-detection.

For the validation of these priorities see “Testing machine design to the priorities”.
System Level requirements

The system level requirements consist of 3 parts. These 3 parts are the USE-cases, the safety properties and the user constraints.

#### USE-cases

There are 6 USE-cases, which are described below.

##### Starting the machine

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | The machine operator starts the machine, machine parts go to their initials state and the machine starts sorting. |
| Postconditions |  | The machine starts the sorting process. |
| Preconditions |  | - |
| Trigger |  | es user is requi discs that arethe machine has stopped running. The resting State is also the state from which you can start thBooting the machine / finished the abort or start/stop routine |
| Basic Flow: | - Machine puts devices in their initial state. - The user presses the START/STOP button |  |

##### Stop the machine

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | The machine is waiting for the current process to end before it is send into an inactive state. |
| Postconditions |  | The machine is sent into an inactive state with no process interrupted. |
| Preconditions |  | The machine is running. |
| Trigger |  | The START/STOP button is pressed. |
| Basic Flow: | - The machine finishes sorting the disks currently in the machine The machine enters an inactive state and will not take any more disks form the storage* unless the START/STOP button is pressed |  |

##### Sort unsorted disks

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | The machine sorts the unsorted disks provided into two separate containers based on colour. |
| Postconditions |  | There are no unsorted disks left All sorted disks are in a container based on their colour |
| Preconditions |  | The machine is not already running. |
| Trigger |  | The user provides unsorted disks and presses the “START” button. |
| Basic Flow: | - An unsorted disk is moved to the colour detector The machine decides to which of the two containers the disk needs to be moved The machine moves the disk to the designated container The machine repeats step 2 through 4 until all disks have been sorted The machine pauses within 4 seconds |  |

##### Abort the process

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | The machine should immediately stop doing anything. |
| Postconditions |  | The machine stopped running and is ready to start again. |
| Preconditions |  | The machine is sorting discs. |
| Trigger |  | The use wants to immediately stop the machine. |
| Basic Flow: | - The machine stops transporting the discs. And doesn’t put any more discs on the transporting mechanism. The user is required to remove all discs that are neither in the container unit nor sorted. When the user removed all unsorted discs that were not in the container unit he presses the START/STOP button. |  |

Booting of the machine

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | The machine will prepare to start the program. And do the required actions. |
| Postconditions |  | The machine is ready to get instructions of the user. |
| Preconditions |  | The machine is off. |
| Trigger |  | N/a |
| Basic Flow: | - Connect the PP2-board to the pc. Plug the pp2-board in to the power socket. Start the debugger Connect the pp2-board using the debugger. Load the program into the debugger. Run the program. |  |

Shutting down the machine

| Primary Actor |  | Machine operator (student or teacher at Tu/e) |
| --- | --- | --- |
| Scope |  | A sorting machine |
| Brief |  | User unplugs the power supply and disconnects the processor from the PC and the machine. |
| Post conditions |  | The PC can be used for other things and the processor and machine can be stored separately. |
| Preconditions |  | Everything is in its initial state or the machine has stopped. |
| Trigger |  | N/a |
| Basic Flow: | - Unplug the power supply of the machine. Unplug the power supply of the processor. Disconnect the processor from the machine. Disconnect the PC from the processor. |  |

### User Constraints

- Before the start button is pressed, the user is required to place all discs to be sorted in the container unit

- While the machine is running the user is not allowed to move the machine or touch anything except the buttons.

- When the abort button is pressed or the machine has been shut down, the user is required to remove all discs that are neither in the container unit nor sorted.

### Safety Properties

- After pressing an emergency button, within 50ms there should be no moving part in the machine

- If all disks are sorted the machine should stop within 4 seconds.

- After the start-up of the machine, the assembly program should not stop until the machine is shut down.

- The outputs connected to the h-bridge may never be powered on at the same time.

- The outputs connected to the motors should never output more than 9 volts

#### Explanation of Safety Properties

- When there is an emergency it is important that whatever is going wrong will not get worse. One of the ways this can happen is for instance that someone's finger gets stuck, to minimize damage to this finger the machine should stop quite fast. After discussion we decided 50ms would be a reasonable maximum stop time as it whatever is going wrong will not get worse in 50ms.

- To minimize electricity usage we think that the machine should not keep running while there are no disks in it.

- If the assembly program stops while the machine is still running, we can no longer control the machine. We can for instance no longer detect when the emergency button is pressed, meaning we cannot guarantee safety property #1.

- The H-bridge should never have two inputs powered on at the same time. Because then you create a short circuit.

- According to the project guide this is the maximum voltage the motors are certified to work with.

### Design Decisions

The way we approached the design of the machine is by separating the machine into multiple parts. Those parts exist out of: the feeder, the transportation mechanism, and the sorter.

#### The Feeder

The feeder has as objective that it needs to somehow get the disks from the container onto the conveyor belt. This is needed for the use case “Sort unsorted disks”.

For the design of this feeder we had two competing designs. Both use the two hollow tubes stacked as a container. We chose to do this because they are completely reliable in containing the disks and because a new disk simply falls out if the bottom one is removed, they are very fast. Because the container is made off two big parts and some small parts to make them stack, the container is also very robust. It’s quite easy to put the disks into the big hole at the top, so user accessibility was very high. In short, the first solution that came to mind scored extremely high on all priorities and we looked no further.

The first design for the feeder consist of 3 important parts. First you have the container. The container drops a disk, which is then pushed onto the conveyor belt using a cam. A wall to the left of the container makes sure the disk is pushed up and not to the left.

Our second feeder design also consisted of a block that pushes the disk. To make this block  move a lever attached to a wheel is used. Rotating the wheel makes the block move back and forth, pushing disks onto the conveyor belt.

Both designs correctly implemented the use cases. To test which one would be better we build both and tested them. They scored the same on almost all top priorities. They were both completely reliable for instance. There was also no difference in speed, both would push a disk onto the conveyor belt with every turn of their wheels. Both did not hinder the user, so the good user accessibility of the container was unchanged. When we came to the last three priorities there were some differences making us choose the first design: It was easier to build, used less parts and was a lot more compact.

# The Transportation and Scanning

When considering the transportation method we had a 3 main ideas. The first one was that we used a short conveyor belt. The second idea was about a long conveyor belt. And the last idea used a turning wheel and 2 conveyor belts. All these ideas included a conveyor belt because that was required.

The thought behind the short conveyor belt was that in the feeding mechanism would push the discs hard enough so that we could put the sensors on that part and to have a small but conveyor belt to transport the discs. The conveyor was short because nothing needed to happen on it. Thus it would only be there because it was a requirement. To us it seemed a bit useless to not do anything on the conveyors belts. So that was when the second arose.

The second idea had a long conveyor belt to put the sensors on. And also a part of the separating mechanism. The conveyor belt would limit how fast the machine can run but all the actions would happen on the conveyor belt so that time wouldn’t be wasted. It also isn’t that hard to create a long conveyor belt so we kept the idea in mind.

Our final idea was that there would be some sort of wheel with separate compartments for discs in the centre which would rotate and put discs on to two different conveyor belts. Each conveyor belt led to a storage unit of the sorted discs. The problem with this idea was that it would be hard to prevent the discs from spinning out of the compartments when they shouldn’t while still being able to let the discs go out when they had to. Because we couldn’t get it to work the idea was dropped and we went back to the idea about a long conveyor belt.

We were capable of realizing the of the long conveyor belt. But during the build of the conveyor belt we noticed that it would not be tight enough around the gears. Thus we tried to remove a small part of the belt. But this still didn’t have to effect we hoped for. So we added a third gear in the middle which tightened the belt to an acceptable state.

The conveyor belt was still far from perfect because it would tilt at certain points and the discs could fall off. So to prevent it we build 2 walls around the belt. On the first part they are low because the low walls were more robust than the high walls and for the user it is easier to access the discs on the conveyor belt. The high walls have been secured using 4 pillars because that made it robust enough to make sure they didn’t break. The walls had to be high because we needed to put a set of sensors on it.

Those sensor had to be above the conveyor belt. They also needed to be at an angle to work properly. That was required else the sensor wouldn’t be able to check if the disc was black or white.

The other set of sensors didn’t need to be place at an angle thus they were simply put on each side of the conveyor belt. This set of sensor would then be capable to scan if there was a disc on that spot of the conveyor belt. This sensor is need to time at which moment the other set of sensor had to check the colour of the disc. And it is also used to check if there are any more discs left to scan.

# The sorting mechanism

For the mechanism that does the actual sorting we chose between a couple of different designs. These designs are listed and explained below.

The first, and most simple design was to use just one conveyor belt that would move left or right based on the colour of the disks. This design is listed under the use of the conveyor belt above, this is why I will not describe it again.

The second design is a slight improvement on the first one where we would use a second, shorter, conveyor belt to do the sorting. This design would place the two conveyor belts in a T-shape with the colour check done on the first one, after which the second conveyor belt moves left or right. We considered this design an improvement on the first one because the second conveyor belt could be made much shorter. This means that the design can sort faster than the single conveyor belt one.

The second conveyor belt was faster than the first design with only one belt, however we soon realized that we could do this even faster. By removing the second belt and replacing it with a seesaw that could be angled to face one of the two sorted containers, we could increase the speed even more. Since the disk would essentially be sorted the moment it reached the end of the conveyor belt. This would be a great design, was it not for the fact that the seesaw required a lot of height. In fact, the entire machine looked like it was placed on stilts, requiring us to use lots of parts and having a lot of wasted space underneath. This design could do it faster at the cost of requiring more space than any of the others.

While the use of a seesaw sped up the sorting process, it also took a lot more space, so we went back to the drawing board and discarded this idea. Instead coming up with a wedge that would be slide onto the conveyor belt from the side whenever a disk of a certain colour is detected. This would then allow the conveyor belt to push the disk against the wedge making a roughly 45° angle thus pushing the disk of the side of the belt and into the collection box. The second colour could just continue while the wedge was pulled back and off the end of the belt. This means that the design cuts off part of the machine at the end and allowing us to make the machine lower than before.

We liked the idea of letting the conveyor belt doing the sorting by placing a wedge in the way, but after some thinking we realized that it could be done both faster and more compact. The trick was to change the direction in the wedge moves from horizontal to vertical. Doing so moves the entire mechanism, aside from the wedge itself, in an upright position pushing it very close to the machine. Aside from saving space, this also allowed the wedge to move much less, since it only has to move just over 1cm above the conveyor belt rather than move all the way over it to the side. This final design does not sacrifice any reliability from its predecessors while being the fastest. It also takes by far the lowest amount of floor space, characterized by the fact that this final design including this sorting mechanism is our only design that fits on only one of the two provided floor plates. For these reasons we believe this design for the sorting mechanism to be the best.

# Machine interface

# The feeder

The motor for the feeder turns a clam. With that motor turning clockwise the disc, which is on the surface in front of the clam, will be pushed off the surface and on to the conveyor belt. To make sure the engine runs clockwise the minus has to be connected to the connection closest to the spot where 6V is marked. We connect this engine to the 3rd output of the pp2-processor.

# The position sensor

The way a position sensor is set up us by using a lens lamp and a phototransistor. The lens lamp will be shining in the direction of the phototransistor. The light from the lens lamp makes the phototransistor send a signal to the pp2-processor. If a disc comes in between the lens lamp and the phototransistor then there won’t shine any light at the phototransistor and thus it won’t send a signal to the pp2-processor. The phototransistor is connected to the 8th input of the pp2-board. The phototransistor is polarized and thus it is important that it is connected correctly. The correct way to connect is with the ground to the connection closest to the white spot on the phototransistor. The lens lamp isn’t polarized and does not move in any direction and thus it doesn’t matter in which connection the ground is. The lens lamp is connected to the 2nd output of the pp2-processor.

# The black white detector

The black white detector uses the same components as the position sensor but they are implemented in a different way. The way in which the colour is detected is by the reflection of light on the disc. Because white discs reflect light very well the phototransistor does pick up some light and thus sends a signal. Black disc on the other hand do not reflect enough light to let the phototransistor pick it up. Thus a white disc can be detected if the sensors are placed in the correct way.

To make sure the phototransistor picks up only the reflected light a cap is placed over it with a hole in the middle. So only light from in front of it will influence the phototransistor. But to make sure that the reflected light can pass through that hole the sensor must be placed at an angle. The reflected light, which is detected by the phototransistor, is at its strongest when the lens lamp is also placed at an angle.

We connected the lens lamp in the same way as the lens lamp of the position sensor only now to the 6th output of the pp2-processor. The phototransistor is also connected as described in the position sensor only now to the 3rd inputs.

| Output | The range/type of the value |
| --- | --- |
| Start/Stop button | Boolean value |
| Abort button | Boolean value |
| Push button(sensor) | Boolean value |
| Colour detector | Boolean value |
| Position detector | Boolean value |
| Timer | Values range from seconds to clock ticks |

I/O tables

Outputs

Inputs

| Input | The range/type of the value |
| --- | --- |
| Lens lamp 1 | Boolean value |
| Lens lamp 2 | Boolean value |
| Conveyer engine | Between 6 and 9 V (Volts) while running 0 V when not running |
| Feeder engine | Between 3 and 7 V while running 0 V when not running |
| Sorter engine | Between 6 and 9 V while running 0 V when not running |
| Display | Integer value, positive |

Picture

Out 0-1

Out 3

Out 7

In 6

Out 6

Out 2

Disp

In

1 0

Out 0-1

Out  7, 6,       3, 2

In 7, 6, 5

In 7

In 5

# The Sorter

The divider uses a so-called “H-bridge” to move up and down. We use output 0 and output to control the H-bridge, which in turn controls the motor moving the divider. We connect the ground of the H bridge with the output 0 to the 6-side of the motor. Now when we power up output 0 the divider will move up. When we power up output 1 the divider will move down. Output 0 and output 1 are never allowed to be on at the same time, which is also stated in the safety properties. We want to move the divider as fast as possible so we always use the maximum allowed voltage of 9 volts. To detect when the divider is in its upmost position we use a push sensor. When the PP2 detects that this push sensor is pressed we immediately cut the power to output 0. We do not detect when the divider is at the bottom, because as soon as the push sensor is not pressed then there isn’t enough space for a disc to go underneath. Thus we simply power on the motor for a set amount of time. This time should be enough to make it move to the bottom but not low enough to interfere with the conveyor belt.

# The buttons

The button that is used to start/stop the machine will be button 0. The button to abort the machine will be button 1.

# The conveyer belt

The conveyer belt uses 5 gears of which only 3 touch the conveyer belt. 2 of those 3 gears are used to make sure the conveyer belt is horizontal and the third one is used to make the conveyer belt turn. The third gear is connected to a metal rod. On that metal rod another gear is connected and that gear will be turned using the gear which is connected to the engine. Because we have those gears in between the direction in which the engine turns has to be counter clockwise. Then the conveyer belt does turn clockwise and the discs will be moved in the right direction. To let the engine turn clockwise we have to connect the ground to the connection closest to the 9V. This engine is connected to the 3rd output.

# System Validation and Testing

# Validate High level specifications

Our high level specifications are correct, because in the exercise it is said that a sorting machine for black and white discs should be made. And it also is said that we need at least one conveyer belt.

# Validation System Level Requirements

The high level specification defines the basic flow of the use-cases, user constraints and safety properties. At the same time, we validate the System Level Requirements through the high level specification. “Sort unsorted discs” is correct, because the high level specification mentions that the machine should sort discs. Aborting the process happens because in every machine something could go wrong and thus it needs to be able to be stopped at any point in time. “Starting the machine” and “Stopping the machine” are actions which are also needed for machines because else you couldn’t make them stop or start doing what they are supposed to do. “Booting up the machine” and “shutting down the machine” is required, because the disc sorter has to be turned on and off, in order for it to fulfil its purpose.

Before the start buttons is pressed the user is required to place all discs to be sorted in the container unit. The discs should be placed in the container, so that the machine is able to sort the discs.
While the machine is running the user is not allowed to move the machine or touch anything except the buttons. If the user makes contact with either the conveyor belt or the discs while they’re on the conveyor belt, the machine might not be able to separate the discs correctly.
When the abort button is pressed or the machine has to be shut down, the user is required to remove all discs that are neither in the container unit nor sorted. The user is supposed to do this, so that the machine will be able to restart the sorting process with a new disc.

After pressing an emergency button, within 50 ms there should be no moving parts in the machine. The machine should immediately abort its current process, according to the high level specification, although this is not realisable. Therefore, this is set to be within 50 ms.

According to the High level Specification the machine should stop sorting if there is no more disk signalled after 4s. We made this into a safety property, because a running machine with no use is only going to possibly harm people getting in contact or the machine itself.

According to what the high level specification offer, there is nothing that could stop the assembly program as long as the code is correctly written for this purpose, we don’t consider accidents and flaws, the only way for the program to end is by powering off the machine.

The outputs connected to the h-bridge may never be powered on at the same time. If this happens, the PP2 processor short circuit, and the machine won’t work anymore.

# Validation Priorities to SLRs

Reliability:

The use-cases describe how we want to sort multiple coloured disks, because we want the sorting to be done as accurately as possible we chose reliability as one of our priorities.

Accessibility:

The use-cases describe that the user has to remove all disks from the machine after the “ABORT” button is pressed. Because of this we want to make the machine somewhat open, so the user can remove the disks with relative ease.

Speed:

The use-cases describe how we want to sort multiple coloured disks, because we want the sorting to be done as fast as possible we chose speed as one of our priorities.

Robustness:

The use-cases describe that the user has to remove all disks from the machine after the “ABORT” button is pressed. For this reason we want the machine to be fairly durable so that the user does not easily damage it. Additionally, since the machine contains a number of engines and moving parts, it will be vibrating ever so slightly. These vibrations should also not cause any damage to the machine leading to our priority of robustness.

Amount of space:

This priority does not have a clear relation to our SLRs, however, we believe that a small machine capable of accomplishing the same task is generally better than a larger version. This is because the machine has to be stored or placed somewhere, leaving you with more space for other machines. This is why we chose for minimizing floor space as one of our priorities.

Difficulty of building:

This priority also does not have a clear relation to our SLRs, but this would make our job as builders easier. It would also allow for greater rates of production of the machine. For these reasons we chose difficulty of building as one of our priorities.

Amount of parts:

This priority also does not have a clear relation to our SLRs. A lot of parts, though, would make our machine more expensive and harsher on the environment, leading us to make the amount of parts one of our priorities.

Because the priorities “Amount of space”, “Difficulty of building” and “Amount of parts” have no clear relationship to the SLRs we chose to put them on the bottom of our priority list.

# Testing machine design to the priorities

1. Perform a test with alternating black and white discs to test the moving of the divider multiple times and check that the discs are sorted right and all discs were sorted.

2. Check if it sorts 10 discs within 30s with a load of white discs, black discs and alternating black and white discs

3. Let the machine perform a run without pushing buttons and with pushing the abort button while running and check if nothing breaks.

4. Look at points in the machine where a disc could get stuck and check if you can access the disc to remove it.

5. Check if the machine fits on 1 floorboard of the Fischer Technik.

6. Check if you can build the machine within 1.5 hours with 2 people.

7. Check if there are any parts without a function.

## Software Specification

In the Software Specification phase, we give an as accurately as possible description of the required behaviour of the PP2, without describing how this is achieved, and a UPPAAL model of this behaviour. In order to do this, we translate the system level requirements to a high level specification of what the software controlling the physical machine should do.

### Inputs and Outputs

| Inputs | The range/type of the value | Abbreviation |
| --- | --- | --- |
| Start/Stop button | Boolean value | In 0 |
| Abort button | Boolean value | In 1 |
| Push button(sensor) | Boolean value | In 5 |
| Colour detector | Boolean value | In 6 |
| Position detector | Boolean value | In 7 |
| Timer | Values range from seconds to clock ticks | Tim |

Inputs

The Start/Stop and Abort buttons speak for themselves. They are either pressed or not pressed.

Push button(sensor): the sorter touches the push sensor or doesn’t touch it, to detect the sorter’s position.

The position sensor and colour detector are either on or off.

Timer

The timer is a count-down timer that is set to a certain value and runs at a frequency of 10 kHz. All given times were calculated by taking the average time of ten measurements, using 50 to 60% of the Potentiometer on the PP2 board. Thus, the sorting mechanisms are faster in reality. The input of a timer is set to a defined value or not set.

TEnd is the moment of termination of the timer, so when the timer reaches zero.

Motor Down is defined as the time it takes for the engine of the sorter to move the sorter from the lowest point to the highest point, until sorting mechanism touches the push sensor. This takes 0.30 seconds.

Motor Up is the state of the sorter moving from the highest point to the bottom of the engine sorter. Since the engine sorter for Motor Down and Motor Up have the same voltage, this will take 0.30 seconds as well.

Sort is the amount of time it takes for a disc to be transported from the black/white detector to the end of the conveyor belt, which is measured to be 0.85 seconds.

Belt is the period that a disc travels from the feeder to the end of the conveyor belt, until the disc reaches the tray for black discs. This action takes 2.0 seconds.

Tic is defined as one clock tick of the PP2. A clock tick is incredibly fast.

Outputs

| Output | The range/type of the value | Abbreviation |
| --- | --- | --- |
| Lens lamp 1 | Boolean value | Out 2 |
| Lens lamp 2 | Boolean value | Out 6 |
| Conveyer engine | Between 6 and 9 V (Volts) while running 0 V when not running | Out 7 |
| Feeder engine | Between 3 and 7 V while running 0 V when not running | Out 3 |
| Sorter engine | Between 6 and 9 V while running 0 V when not running | Out 0-1 |
| Display | Integer value, positive | Disp |

Lens lamp position and lens lamp sorter are the lamps that make up part of the sensors and can be turned on or off.

The conveyor and feeder engines respectively move the conveyor belt and the feeder. They are either on or off.

Hbridge0 indicates whether the sorter moves up or not. On the other hand, whereas Hbridge1 shows that the sorter moves down or halts.

The display shows the state that the machine is currently in. Depending on the available time, we might or might not implement this.

The Timer start output is the same as the Timer input, except that the timer counts down.

#### Validation of “Inputs and Outputs”

We see that the inputs and outputs of Software Specification are correct. The inputs of Machine Design should be equal to the outputs of Software Specification, which they are.

## Relations

## Lens lamp of the black white detector

The lens lamp of the black white detector will be on when the machine is sorting. Thus the lens lamp will react to the input of the “START/STOP” button and the “ABORT” button. The lens lamp will go on when the machine is in resting state and the “START/STOP” button is pressed and it will go off when the “ABORT” button is pressed while the machine was running.

## Lens lamp of the position sensor

The lens lamp of the position sensor reacts only to the “START/STOP” button and the “ABORT” button. The lens lamp will be on after the “START/STOP” button is pressed and the machine is in its resting state. If at any other point in time the “ABORT” button is pressed it will go off. When the “START/STOP” button is pressed and the machine is running then the lens lamp also goes off.

## Engine of the conveyor belt

The engine of on the conveyor belt only reacts to the input of the “START/STOP” button and the “ABORT” button.  The engine will start when the machine is in its resting state and the “START/STOP” button is pressed. If however the “START/STOP” button is pressed and the machine is not in its resting state then the machine will stop after it completed its current cycle.  Whenever the “ABORT” button is pressed the engine stops within 50ms.

## Engine of the feeder

The engine for the feeder also only reacts to the input of the “START/STOP” button and the “ABORT” button. This engine  also starts when the machine is in its resting state and the “START/STOP” button is pressed. If however the machine is running then the engine will stop. When the “ABORT” button is pressed the engine stops within 50ms.

## Engine for the sorter

When the machine is running the engine of the sorter reacts to inputs of the colour detector, the push sensor and the timer. When a signal is received from the colour detector the engine pushes the sorter up, the engine then waits until the timer gives a signal to go down again after it let the discs through, it knows when it is in the correct “up” position from the push sensor . If the “START/STOP” button is pressed when the machine is in its resting state, then the sorter will wait for a signal from the timer that marks the end of the current cycle. If at any time the ““ABORT”” button is pressed, the sorting mechanism is to stop within 50ms.

## Display for counting

The display output depends on how many times the colour detector detects a white disc and how many times a disc passes the position sensor without the colour detector detecting it.

In the initial state the counters get reset.

## Validation of “Relations”

The relations between the inputs and outputs can be validated with the input/output tables. For all inputs, we have outputs. These outputs depend on one or more inputs, which is described in the Relations.

# Design Decisions

Feeder

The feeder in constantly on because of priority 2, speed, mentioned in the Machine Design document. Another reason is that there's a turning part that needs to spin through to get to its initial position to be able to deposit discs again.

Lens lamp position

We chose to have the lens lamp for position sensor constantly on, because it's easier to code resulting in spending less time on it. The optimization is minimal if we would turn them off every time there's a gap between discs, because of the feeder being quite fast in depositing the next disc.

Conveyor belt

The conveyor belt is constantly running, because the feeder is constantly pushing discs onto the conveyor belt. This goes hand in hand with our second priority, which is speed.

Lens lamp colour

Like with the position sensor, it's easier to code that it is continuously on. The light being off if it's possible, would again be a minimal improvement, because the gaps between discs being pushed on the conveyor belt is the same as with the black white detector.

Push button

We use the push button, because of priority 1, correctness, to know if the sorter arm is at its highest point. We need to know this, because we need to know when to stop the motor making the sorter arm going up.

# Description of States

Initial_state

In the initial state the machine starts calibrating the sorting mechanism by moving it up.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 0 |
| Lens lamp sorter | 0 |
| Engine conveyor | 0 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 0 |

Calibrate_Sorter

In the calibrate sorter state the sorting mechanism moves down until it is just above the conveyor belt.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 0 |
| Lens lamp sorter | 0 |
| Engine conveyor | 0 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 1 |
| Display | 0 |
| Timer start | 0 |

Resting_state

In the resting state the sorting machine is at rest and waiting for the user to press the START/STOP button.

| Outputs | Value for output |
| --- | --- |
| Lens lamp | 0 |
| Lens lamp | 0 |
| Engine conveyor | 0 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 0 |

Running_state

In the running state the sorting mechanism, the conveyor belt, the position detector, and the colour detector are turned on.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 2 s + Belt |

Running_Wait

In this state a disc has been detected and that disc is moving along the conveyor belt to the sorter.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 2 s + Belt |

Running_Timer_Reset

In this state a new disc was detected and the timer has been reset.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 2 s + Belt |

Motor_Up

In this state the motor of the sorter is moving up until it hits the push button.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 1 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Sort |

Motor_Up_Stop

In this state the motor of the sorter is moving up until it hits the push button. And the machine has to stop because the start stop button was pressed.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 1 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Sort |

Motor_Down

In the Motor_Down state, the sorter is moved down.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 1 |
| Display | 0 |
| Timer start | 0 |

Motor_Down_Stop

In Motor_Down_Stop, the sorter is moved down, after the start/stop button has been pressed.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 1 |
| Display | 0 |
| Timer start | 0 |

White_Wait

In this state the machine waits until the colour detector has detected a white disc.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Sort |

White_Wait_Stop

In this state the machine waits until the colour detector has detected a white disc, after the START/STOP button has been pressed.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 1 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Sort |

Running_Timer

Running_Timer is the state that sets the interrupt timer to make sure the machine stops after the current cycle.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Belt |

Motor_Up_Timer

Motor_Up_Timer is the state that sets the interrupt timer to make sure the machine stops after the current cycle.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Belt |

White_Wait_Timer

White_Wait_Timer is the state that sets the interrupt timer to make sure the machine stops after the current cycle.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Belt |

Motor_Down_Timer

Motor_Down_Timer is the state that sets the interrupt timer to make sure the machine stops after the current cycle.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | Belt |

Aborted

Aborted is the state where the machines goes to if the abort button is pressed, the machine has come to a halt.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 0 |
| Lens lamp sorter | 0 |
| Engine conveyor | 0 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Display | 0 |
| Timer start | 0 |

Running_Stop

Running_Stop gives the same outputs as the Running state, the only difference being a running timer in the stop process.

| Outputs | Value for output |
| --- | --- |
| Lens lamp position | 1 |
| Lens lamp sorter | 1 |
| Engine conveyor | 1 |
| Engine feeder | 0 |
| Hbridge0 | 0 |
| Hbridge1 | 0 |
| Engine sorter | 0 |
| Display | 0 |
| Timer start | Belt |

## Validation of “Description of States”

To validate the states we will look at the USE-cases again to see if every USE-case is implemented. To do this we look at the basic flow and trigger of every use case and see what states we use to realize this.

We also validate the states to the relations. For every USE-case we looked at what states would be necessary to achieve it.

### Starting the machine

Preconditions: -

Trigger: Booting the machine / finished the abort or start/stop routine

Postconditions: The machine starts the sorting process.

| Basic Flow | State | Explanation |
| --- | --- | --- |
| Before Trigger | Any State | It does not really matter which state the machine is in before the trigger |
| After Trigger | Initial State | Initial state is the first state, so after booting the machine we will be here. Finishing the abort or start/stop routine will also end in the initial state |
| - Machine puts devices in their initial state. | Initial State + Calibrate Sorter + Resting State | The only thing that needs to be put into an initial state is the sorter mechanism. In initial state the machine moves the sorter up until it touches the push button. It then transitions to Calibrate Sorter where it starts moving down. After a set amount of time it will stop moving the sorter and transition to the resting state. This way we know exactly where the sorter is positioned |
| - The user presses the START/STOP button | Running State | From the Resting State the transition to the running state is pressing the START/STOP button |
| Postconditions | Running State | The running state is the start of the sorting process |

### Stopping the machine

Preconditions: The machine is running.

Trigger: The START/STOP button is pressed.

Postconditions: The machine is sent into an inactive state with no process interrupted.

| Basic Flow | State | Explanation |
| --- | --- | --- |
| Preconditions | Not initial state, Calibrate Sorter or aborted | When the machine is not in any of these states it is running. |
| After Trigger | One of the (greenblue) Timer statesaria the machine is not in any of these states it is running.s means they do not use states. This also means we can'sitioned. th | When the START/STOP is pressed the machine transitions to a timer start state, which starts a timer and stops the feeder mechanism. |
| - The machine finishes sorting the discs currently in the machine | One of the sorting states | While the timer is running the machine keeps sorting. The timer is the time it takes for the conveyor belt to make a complete rotation, guaranteeing there are no more discs on the belt. |
| - The machine enters an inactive state and will not take any more discs form the storage* unless the START/STOP button is pressed. | Initial State + Calibrate Sorter + Resting State | After going through the initialize process we go back to the resting state, which waits on the START/STOP button. |
| Postconditions | Resting StateRstiresting state, which waits on the START/STOP button. timer is the time it takes for the conveyor belt to make a complete ro | Resting state in an inactive state and we finished the sorting process. |

### Sort unsorted discs

Preconditions: The machine is not already running.

Trigger: The user provides unsorted discs and presses the “START” button.

Postconditions: There are no unsorted discs left, all sorted discs are in a container based on their colour.

| Basic Flow | State | Explanation |
| --- | --- | --- |
| Preconditions | Resting State | The program first initializes and then waits for the user to press that start button. This waiting happens in the Resting State. In the resting state the machine is not running |
| After Trigger | Running State | Pressing START/STOP is the input to transition to the running state |
| - An unsorted disc is moved to the colour detector | Running Wait + Running Timer Rest | When moving to the colour detector it will have to pass the position Sensor which is the input to move to Running Wait, the disc is then still in front of the position sensor so the program moves to Running Timer Rest |
| - The machine decides to which of the two containers the disc needs to be moved | Running Wait + Running Timer Rest OR Motor Up + White-Wait | Depending on whether the disc is white or black the sorter either needs to move down or keep its down position. If it keeps its down position it should just keep checking for an unsorted disc and when it detects one it will move to Running Timer Rest If it needs to move up the colour detector will detect a white disc and therefore transition to Motor Up. Moving the sorter up will trigger the pushButton, which is the input to transition to White-Wait |
| - The machine moves the disc to the designated container | Running Wait + Running Timer Rest OR Motor Down + Running Wait | If the sorter did not detect a white disc we are still waiting like in basic flow 2. If it did detect one then while the disc is moving to the designated container the sorttimer will count down making the machine transition to Motor Down |
| - The machine repeats step 2 through 4 until all discs have been sorted | - |  |
| - The machine pauses within 4 seconds | Initial State + Calibrate Sorter + Resting State | If there are no discs anymore the machine will stay in Running Wait waiting for the timer interrupt which will come within 4 seconds, making the machine transition to initial state. There it will reset the sorter and transition to the resting state |
| Postconditions | Resting State | We repeated the sorting step until all discs where sorted, meaning all discs are now sorted |

### Abort the process

Preconditions: The machine is sorting discs

Trigger: The user wants to immediately stop the machine.

Postconditions: The machine stopped running and is ready to start again.

| Basic Flow | State | Explanation |
| --- | --- | --- |
| Preconditions | Every that is not initial state, Calibrate Sorter, resting state or Abortedcdlsls | All other states are states in which discs are being sorted |
| After Trigger | Aborted | Every state (apart from the one mentioned in before trigger) have a line to abort with Abort as input |
| - The machine stops transporting the discs. And doesn’t put any more discs on the transporting mechanism. | Aborted | Because the machine is now in the abort state, which has all outputs set to 0, nothing will be moving. |
| - The user is required to remove all discs that are neither in the container unit nor sorted. | Aborted | The machine will remain in Abort until the user presses START/STOP. This means everything is stopped and the user can safely remove all discs |
| - When the user removed all unsorted discs that were not in the container unit he presses the START/STOP button. | Initial State + Calibrate Sorter + Resting State | Pressing the START/STOP button is the input for the transition to Initial State There it will reset the sorter and transition to the resting state |
| Postconditions | Resting State | We are in the resting state, so the machine has stopped running. The resting State is also the state from which you can start the machine again |

Booting of the machine and Shutting down the machine do nothing with our software. This means they do not use states. This also means we can’t validate those USE-Cases here.

# State transitions

| Current state | Input | Input value | Next State |
| --- | --- | --- | --- |
| Initial | Push | 1 | Calibrate_Sorter |
| Calibrate_Sorter | Push | 0 | Resting |
| Resting | StartStop | 1 | Running |
| Running | Timer | TEnd | Initial |
| Running | PositionSensor | 0 | Running_Wait |
| Running | Abort | 1 | Aborted |
| Running | StartStop | 1 | Running_Timer |
| Running_Wait | Timer | TEnd | Initial |
| Running_Wait | PositionSensor | 0 | Running_Timer_Reset |
| Running_Wait | ColorDetector | 1 | MotorUp |
| Running_Wait | StartStop | 1 | Running_Timer |
| Running_Wait | Abort | 1 | Aborted |
| Running_Timer_Reset | Tick | 1 | Running_Wait |
| Running_Timer_Reset | Abort | 1 | Aborted |
| MotorUp | PushButton | 1 | WhiteWait |
| MotorUp | StartStop | 1 | Motor_Up_Timer |
| MotorUp | Abort | 1 | Aborted |
| WhiteWait | StartStop | 1 | White_Wait_Timer |
| WhiteWait | Abort | 1 | Aborted |
| WhiteWait | Timer | SORT | MotorDown |
| MotorDown | StartStop | 1 | Motor_Down_Timer |
| MotorDown | Abort | 1 | Aborted |
| MotorDown | Timer | Motor Down | Running_Wait |
| Running_Timer | Timer | Tic | Running_Stop |
| Running_Timer | Abort | 1 | Aborted |
| Motor_Up_Timer | Timer | Tic | Motor_Up_Stop |
| Motor_Up_Timer | Abort | 1 | Aborted |
| White_Wait_Timer | Timer | Tic | White_Wait_Stop |
| White_Wait_Timer | Abort | 1 | Aborted |
| Motor_Down_Timer | Timer | Tic | Motor_Down_Stop |
| Motor_Down_Timer | Abort | 1 | Aborted |
| Motor_Up_Stop | PushButton | 1 | White_Wait_Stop |
| Motor_Up_Stop | Abort | 1 | Running_Stop |
| Motor_Up_Stop | Timer | Timer Interrupt | Initial |
| Motor_Up_Stop | Abort | 1 | Aborted |
| White_Wait_Stop | Timer | SORT | Motor_Down_Stop |
| White_Wait_Stop | Abort | 1 | Aborted |
| White_Wait_Stop | Timer | Timer Interrupt | Initial |
| Motor_Down_Stop | Timer | Motor Down | Running_Stop |
| Motor_Down_Stop | Abort | 1 | Aborted |
| Motor_Down_Stop | Timer | Timer Interrupt | Initial |
| Running_Stop | ColorDetector | 1 | Motor_Up_Stop |
| Running_Stop | Abort | 1 | Aborted |
| Running_Stop | Timer | Timer Interrupt | Initial |
| Aborted | StartStop | 1 | Initial |

## Validation of “State Transitions”

The description of our machine states is validated through its representation in the transition table. No state is excluded from being represented in the state transition table, all transitions will have the initial transition state differ from the end state.

# Finite-state Automaton

Blue line means that the trigger for the transition is a clocktick

## Validation of “Finite-state Automaton”

When we were making our finite-state automaton we looked at our state description and made sure that all states were represented, then we used our state transition table to make sure all transitions were correctly implemented.

# UPPAAL model

## Tests done

On the next page is the UPPAAL model. This UPPAAL model has been tested for 2 safety properties. The first one is “ After the start-up of the machine, the assembly program should not stop until the machine is shut down.”. This has been tested using the following property “A[] not deadlock”, and we didn’t have a deadlock. The second safety property which was tested is: “The outputs connected to the h-bridge may never be powered on at the same time.”. This was tested using the following property “A<> !(hbridge0==1 && hbridge1=1)”. This one was also correct.

## Validation of “UPPAAL model”

All transitions which exist in the UPPAAL model also occur in the Finite State Automaton. And  the same action has to be performed to take that transition. Also all states of the Finite State Automaton occur in the UPPAAL model. The states of the UPPAAL model also have the outputs in them. The states of the Finite State Automaton do not have the outputs in them. Thus we validate the values of the outputs, which are in the states, to the description of the states.

# Software Design

In the Software Design phase, we present a Java program that realises the functions specified in the Software Specification document. This program is an intermediate step towards writing the PP2 code that controls the sorting machine.

Coding Standards

The java pseudo code follows the Google Java Style. 
Source to Google Java Style: https://google-styleguide.googlecode.com/svn/trunk/javaguide.html.

PHP code used in this project follows the Zend Framework Coding Standard for PHP.
Source: http://framework.zend.com/manual/1.12/en/coding-standard.html.

Translating to pseudo java:

The java program starts by declaring the output variables. The names of the output variables will keep their original name, without spaces, in a camelCase form. The variable type will be determined from the Output table.

The inputs follow the same pattern.

Every state is represented as a function, keeping their name in the camelCase fashion, they will be all void functions due to the fact that they do not return anything.

Every state function will run preconditions if any, then check for specific input values using if statements, if an if statement is satisfied, there will be changes to the output values to match the next states output values, also the display is set to output the next states number, and then the next state function is called according to the state transition diagram, if no if  statement is satisfied the current function is recalled.

The program is always looping, consequence of no deadlocks in the state machine as proven by the UPPAAL model test.

Example: Initial -> Calibrate_Sensor

So in this example the function initial is currently running, there are no preconditions to be checked, if the inputs have the desired value, in our case we check if the push button is pressed by the sorter, if so we will have the sorter moved down by activating the sorter motor via having the Hbridge0 variable set to 1. After this we set the display to showcase the number $branchTO where to branch to2 then call calibrateSensor function and if the if  statement wasn’t satisfied we recall initial entering a loop.

## Translating from Java to PHP

The java code was written such that the conversion process to php is as easy as possible.

All variable in java will have the “$” sign added at the beginning of their name to comply with the php standards. The “$” sign has no influence in the java program variable naming, while in php it is mandatory.

- Design decisions for the Java code

In translating our transition table to a Java program we made a number of decisions shaping the code, these decisions are outlined in this section.

We started by looking at our transition table, in this table we had our transitions ordered by the “current state”, the state where the transition starts. Then there were some inputs that could trigger a transition from this state to a number of other states. Because of this we thought it would make sense to write a function for each state, since it would allow our code to essentially be a condensed version of the transition table. Where the code would be ordered by the “current state”, and each state would have a number of outgoing transitions to other states. This resulted in the following blueprint for each of our functions:

void function(){

timerManage();		// The function that manages the outputs and PWM

$temp = getButtons();	// Store the buttons currently being pressed in a temporary variable

if( condition for transition ){	// Do this once for every outgoing transition from this state

Changes required to change to the new statement;

}

function();			// Call on the same function again to recheck the buttons and continue running the machine with timerManage()

}

Then we made an extra function which will be called from each function to do the PWM. This function is called timerManage. This function firstly gets the voltage which the output needs from the array.

This function has a variable called counter which increments each time the outputs have been set. That value is take modulo 12. So it will leave the outputs which need 12 volts on all the time. The reason why the values which need less than 12 volt will be turned off after they have been on for long enough. That goes as follows. First it checks if the engine needs to be on by checking if the voltage it needs is higher than counter. If the output needs to be on then it gets the location of the value in the array. And then does 2 to the power of the location. So now the correct output will be set on. Then the value of 2 to the power will be added to the variable engines. Then after all 7 outputs have been through that loop then it will set the output to the value of engines. So the lights which needed to be on will be on. Now the value of counter will increment each time and take modulo 12.

We also choose to save certain values, which may not be expected to be saved. In this section I will explain why we save the 2 variables. The first one is the variable of the location of the code. This has been saved because then we then we are capable of changing the return address after the timer interrupt. Because when an timer interrupt occurs we want to return to the initial state and the position where we were before. We also saved the original position of the stack pointer for when we come back from the timer interrupt to make sure that we empty the stack. Because there may be some values on the stack from before the timer interrupt. Thus to remove them we set the stack pointer to its original value.

Validation

Validation of java to transition table

Every state is represented by a function. The if statements in that function are the transitions which can occur from that state. The timer interrupt and the abort transitions are not represented as if statements, because interrupts go to a separate state(function). In those if statements the values that have to change are changed. The display will also be updated to the correct number of the state. The function timerManage is called in each state. Because with that function we make sure that the all outputs have the correct voltage.

We checked that all states are represented in the java code by a function. We also checked if they have all the transitions as if statements and that the correct values are changed.

  - Validation of timerManage

Loop invariant:

All elements before the current element of the array have been set on if they had to be on.

Initialize:

We start with the first element. Thus there are no elements before it and the loop invariant holds.

Step case:

If we’re at element k, then according to the loop invariant all elements before k have been set on if they had to be on. Then if k has to be on (value of k>counter) it will be set on else it will stay off. So now the loop invariant holds for the element k+1

Termination:

The loop will terminate when k is greater than 7. Because we do not have any more outputs.

  - Control flow validation

Because the Java code has been validated to the state description and the transition table, which, in turn, have been validated with the UPPAAL model and shown to be correct and in tune with the initial description of the sorting machine. This means that the Java program, being a one-to-one translation of the finite state automaton, also has a correct control flow.

# Software Implementation and Integration

Now we show the data representation and coding standard we chose that is used to write the Assembly Language.

# Java to PHP

The Java to PHP conversion is usually natural, the two languages sharing most syntax but there are some differences we must note down. We are not required to create a class in PHP. The initialization will  differ in PHP from Java, but they share the same core in the end. Also while we have some of the variables initialized globally in Java, in PHP they will be local. Having no class will make the class initialization irrelevant in PHP and that’s why its missing. The later functions in the Java code right after the function TimerManage are included in the PHP code using “include “functions.php”;”. In TimerManage, % operation is replaced by the mod() function. Due to our PHP compiler limitations we are required to use variables as arguments when calling certain functions like for example storeData. The PHP code has been added as appendix 5.

# Validation of Java to PHP

Because of the natural similarity and ease of conversion, the PHP codes correctness can be correlated to its java counterpart, the correctness of the java code was validated in the Validation part of the Software Design.

# Validation of PHP compiler to Assembly

The compiler works in phases. We will go through these phases 1 by 1 to explain how the compiler does its job: compiling PHP-like code to assembly. Throughout the phases the compiler keeps track of the line number of the PHP code it is currently compiling and uses that, when an error occurs, to give information where the error is. The compiler is written in PHP5.6 and uses a command line interface.

## Preprocessing

In the first phase, the input code will be made ready for the next steps. A few things happen in this phase: First the file is read into the memory. The next step is that all comments, newlines and extra spaces are stripped from the file. The file is then split into single lines using the “;” symbol that denotes the end of a line. While doing this the compiler writes the data to two arrays: the data array for everything between “//**DATA**” and “//**CODE**” and the code array for everything after “//**CODE**”. Everything before //**DATA** is ignored. The data array gets compiled immediately.

The preprocessor further removes some special statements that are needed to make valid php such as “global” and changes some shortcuts in their full version. For example $abc++ will be changed into $abc+=1. This ensures that the compiler only needs to be able to handle $abc+=1.

## Splitting

In the second phase the code is split up by function. Every function gets his own array with all the lines that are in that function. The code not inside of a function goes into a separate array.

## Compiling

The third phase is the most important one. It starts by compiling the code that is at the start and not inside a function. While compiling it keeps track of what functions are called and adds these, if they are not already compiled, to the toCompile queue. This helps in making sure there is no dead code, as a function that is never called, will not be compiled. The compiler adds the function “main”, which is the default start point of the code, to the queue and starts processing it.

After compiling the main function it will continue in the next function in the toCompile queue and keep doing this till the toCompile queue is empty.

The compiling itself is not a lot more than a lot of regex and switch statements that look at the input and make an output from that. At the first notion of a variable a register is assigned to it. The code then uses this register in place of the variable. Some more difficult statements, like the function display which displays something, will BRS to premade assembly code that handles that. The compiler keeps track of which segments of the premade assembly code are used.

When the compiler meets an if statement, it saves the code inside it to a new function named “condtionali” where i is the amount of conditionals that have already been seen. It then places this function in the toCompile queue. It also saves the location of the end of the if statement, so it will later know where to return when the if function has ended.

## Combining

After there are no functions left in the toCompile queue, the combining phase starts. In this phase all the functions and the code outside the functions are combined into a single array. This phase also adds the used premade functions at the top and inserts the return statements at the correct position.

## Formatting

The last phase is the last interesting. It goes through the, now compiled code, and formats it. It uses either the length of the longest function name or the number 25 depending on which is larger to insert spaces in front of every line of code in a way everything lines up nicely.

The last step the compiler takes is writing the compiled code to a file and using the assembler provided to create the hex code.

# System Validation and Testing

Finally, we demonstrate that the final product meets its initial requirements, i.e. we prove that the executable code correctly implements the System Level Requirements, and that the implementation doesn’t do more than is expected.

Validation Policy

In our documents we have validated every element of contents in a separate Validation section at the end of the document or near to it.

Machine Design will have at the end of the document a Validation section which includes the Validation of High Level Specifications and the Validation of the System Level Requirements, also adding Validation to Design Priorities.

The Software Specification document will have a Validation section that will contain the validation of the Inputs and Outputs, the Relation of Inputs and Outputs, the Description of States, the State Transitions, the Finite State Automaton and the UPPAAL model.

Software Design will have a Validation section close to the end of the document being afterwards followed by the Program Code. The Validation will contain the validation of the java code to the transition table( from the Software Specification), validation of the timerMange function ( this function needed separate formal proof for its inner loop) and Control flow validation.

The Software Implementation and Integration document will have at the end a Validation section containing validation of the PHP code to java and the validation of the Assembly code to the PHP compiler.

Conclusion

To be completed.

Product to Problem

To be completed.

References

# Process

Design decisions

Work Plan

To streamline the group process we needed a Work Plan. We started this Work Plan with the inventory of the goals and objectives of each phase of the project. For the roles in the group we chose to have them the same as described in /Project Guide Design Based Learning "DBL 2IO70" "Sort It Out"/.

Then we come to the definition of our terms. We chose to have abbreviations of the phases and the tasks. This way we can refer to them without having to waste a lot of space if we mention them multiple times. Also the roles have their abbreviations.

Before we use those abbreviations we first have an inventory of the amount of work and an overview of the main deliverables. The amount of work is given per phase and week in a nifty table. The overview of deliverables contains who's responsible for a certain deliverable and the date and week the deliverable is due.

Then we come to the weekly tables. Tuesday and Friday we have a tutor meeting and we work afterwards till in the afternoon. On Wednesday we have Data Structures in the morning and work on the project afterwards. Those times are included in the tables. Everyone has his column with his role if applicable. For every hour and person it's defined what he will be working on.

With this Work Plan and the collective logbook we're able to have an indication of how much time was spent on each task by each member. If necessary action can be taken based on this indication.

If unforeseen problems arise and the deadline is close, this means we have to work harder. Deadlines aren't easily moved. If someone spends too less time according to the Work Plan it's expected he does his work at home.

Workday

For us, a normal workday is structured as follows: we start each workday with a list of items that needs to be done in order to complete the document for that week. The list is written on the whiteboard that is available in the room. Then members are assigned to a task in consultation. After the completion of a task, it is checked off or removed from the whiteboard, and the member that was responsible for it continues to work on the next item of the inventory until there are no more available assignments. Next, they will help another group member with their duty. This cycle repeats itself whenever we are together. On Wednesday, the document is wrapped up and cross-read. The person that bears the responsibility for the document hands in the current document for feedback when possible. On Friday, the document is updated according to the feedback given by the tutor. Subsequently, the finalised document is cross-read, and handed in by the person responsible for the document.

Problems

There was a problem with the group not functioning as was expected. The logbook indicated that some members contributed less than other members. As a result, other members had to compensate for it by spending more time on the project. Therefore, we decided to address this problem in the meetings and to distribute the workload more evenly.

Work Plan

# Conclusion

Over the course of these past 8 weeks we worked on making a sorting machine and the software that runs it. We did this by going through multiple phases, starting with Machine Design, where we designed the machine itself. Moving to Software Specification, where we created a finite state automaton, then Software Design and Software Implementation and Integration where we respectively designed a pseudo-Java program and then translated that into Assembly for the PP2. While making these documents we validated each part to what we did before to make sure that we made the right decision every time. While the project took a lot of our time each week, we liked doing it, and the end result was very satisfying. We hope that the skills we have acquired over the course of this project, both those for designing and building a product and those for working in a group, will help us in future projects both here in the TU/e and beyond.

# Appendix 2: Java Program

1 /**

2  * Sort of a simulation of the PP2 program

3  * controlling the Fischer

4  * Technik in order to sort black and white discs.

5  *

6  * @author Maarten Keet

7  * @author Stefan van den Berg

8  * @author Rolf Verschuuren

9  * @author Wigger Boelens

10  * @team Group 16

11  * @since 13/3/2015

12  */

13

14

15 class SoftwareDesign {

16     //**@CODE**

17     //inputs

18     int $push, $startStop, $abort, $position,

19             $colour;

20

21     //variables

22     int $state = 0;

23     int $sleep = 0;

24     int $temp = 0;

25     int $location;

26     int $counter = 0;

27     int $engines;

28

29

30     //constants

31     final int TIMEMOTORDOWN = 30;

32     final int BELTROUND = 2000;

33     final int BELT = 1200;

34     final int SORT = 850;

35     final int LENSLAMPPOSITION = 5,

36             LENSLAMPSORTER = 6,

37             HBRIDGE0 = 0,

38             HBRIDGE1 = 1,

39             CONVEYORBELT = 3,

40             FEEDERENGINE = 7,

41             DISPLAY = 8,

42             LEDSTATEINDICATOR = 9;

43

44     public static void main(String args[]) {

45         SoftwareDesign SoftwareDesign = new

46                 SoftwareDesign();

47

48

49         //values for the data segment

50         SoftwareDesign.initVar("outputs", 12);

51         SoftwareDesign.initVar("stackpointer", 1);

52         SoftwareDesign.initVar("offset", 1);

53

54         //store the offset of the programm,this

55         // is used in the interrupt

56         SoftwareDesign.storeData(startofthecode,

57                                  "offset", 0);

58

59         //store the vlue of the stackpointer,so

60         // we can clear the stack

61         // easily

62         SoftwareDesign.storeData(SP,

63                                  "stackpointer",

64                                  0);

65

66         $counter = 0;

67

68

69         //reset outputs

70         SoftwareDesign.storeData(0, "outputs",

71                                  SoftwareDesign

72                                 .HBRIDGE1);

73         SoftwareDesign.storeData(0, "outputs",

74                                  SoftwareDesign

75                                 .LENSLAMPPOSITION);

76         SoftwareDesign.storeData(0, "outputs",

77                                  SoftwareDesign

78                                 .LENSLAMPSORTER);

79         SoftwareDesign.storeData(0, "outputs",

80                                  SoftwareDesign

81                                 .LEDSTATEINDICATOR);

82         SoftwareDesign.storeData(0, "outputs",

83                                  SoftwareDesign

84                                 .DISPLAY);

85         SoftwareDesign.storeData(0, "outputs",

86                                  SoftwareDesign

87                                 .CONVEYORBELT);

88         SoftwareDesign.storeData(0, "outputs",

89                                  SoftwareDesign

90                                 .FEEDERENGINE);

91

92         //start moving the sorter up

93         SoftwareDesign.storeData(9, "outputs",

94                                  SoftwareDesign

95                                  .HBRIDGE0);

96

97         //go to the first state and set the

98         // value for the display

99         SoftwareDesign.$state = 0;

100         SoftwareDesign.initial();

101     }

102

103     //state0

104     void initial() {

105         setStackPointer(

106                 getData("stackpointer", 0));

107         timerManage();

108         //check if the sorter push button is

109         // pressed

110         $push = getButtonPressed(5);

111         if ($push == 1) {

112             //move the sorter down

113             storeData(0, "outputs", HBRIDGE0);

114             storeData(9, "outputs", HBRIDGE1);

115             //update the state

116             $state = 1;

117             //reset sleep for the next function

118             $sleep = 0;

119             calibrateSorter();

120

121         }

122         //loop

123         initial();

124     }

125

126     //state 1

127     void calibrateSorter() {

128         timerManage();

129         //the sorter is now moving down,

130         //and we're waitng for it to reach the

131         // bottom

132         if ($sleep == TIMEMOTORDOWN * 1000) {

133             //stop the sorter

134             storeData(0, "outputs", HBRIDGE1);

135             //update the state

136             $state = 2;

137             //reset sleep

138             $sleep = 0;

139             resting();

140         }

141         //loop

142         $sleep++;

143         calibrateSorter();

144     }

145

146     //state 2

147     void resting() {

148         timerManage();

149         //the program waits for the user to

150         // press the start/stop

151         $startStop = getButtonPressed(0);

152         if ($startStop == 1) {

153             //sleep so we don't go to the pause

154             // immediatly

155             sleep(2000);

156             //power up the lights

157             storeData(12, "outputs",

158                       LENSLAMPPOSITION);

159             storeData(12, "outputs",

160                       LENSLAMPSORTER);

161             //start up the belt and the feeder

162             storeData(9, "outputs", CONVEYORBELT);

163             storeData(5, "outputs", FEEDERENGINE);

164             //set and start the countdown

165             setCountdown(BELTROUND + BELT);

166             startCountdown();

167             //update the state

168             $state = 3;

169             running();

170         }

171         //loop

172         resting();

173     }

174

175     //state 3

176     void running() {

177         timerManage();

178         //check if we need to pause

179         $startStop = getButtonPressed(0);

180         if ($startStop == 1) {

181             //stop the feeder engine

182             storeData(0, "outputs", FEEDERENGINE);

183             //set the timer

184             setCountdown(BELT);

185             //update the state

186             $state = 9;

187             runningTimer();

188         }

189         //check if a disk is at the position

190         // detector

191         $position = getButtonPressed(7);

192         if ($position == 1) {

193             //reset the countdown,because a

194             // disk was detected

195             setCountdown(BELTROUND + BELT);

196             //update the state

197             $state = 4;

198             runningWait();

199         }

200         //loop

201         running();

202     }

203

204     void runningWait() {

205         timerManage();

206         //check if we need to pause

207         $startStop = getButtonPressed(0);

208         if ($startStop == 1) {

209             //stop the feeder engine

210             storeData(0, "outputs", FEEDERENGINE);

211             //set the timer

212             setCountdown(BELT);

213             //update the state

214             $state = 9;

215             runningTimer();

216         }

217         //check if a disk is at the positiond

218         // detector

219         $position = getButtonPressed(7);

220         if ($position == 0) {

221             //reset the countdown,because a

222             // disk was detected

223             setCountdown(BELTROUND + BELT);

224             //update the state

225             $state = 5;

226             runningTimerReset();

227         }

228         //check if a white disk is at the color

229         // detector

230         $colour = getButtonPressed(6);

231         if ($colour == 1) {

232             //move the sorter up

233             storeData(9, "outputs", HBRIDGE0);

234             //update the state

235             $state = 6;

236             motorUp();

237         }

238         //loop

239         runningWait();

240     }

241

242     //state 5

243     void runningTimerReset() {

244         timerManage();

245         //update the state

246         $state = 5;

247         runningWait();

248     }

249

250     //state 6

251     void motorUp() {

252         timerManage();

253         //check if we need to pause

254         $startStop = getButtonPressed(0);

255         if ($startStop == 1) {

256             //stop the feeder engine

257             storeData(0, "outputs", FEEDERENGINE);

258             //set the timer

259             setCountdown(BELT);

260             motorUpTimer();

261         }

262         //check if the sorter push button is

263         // pressed

264         $push = getButtonPressed(5);

265         if ($push == 1) {

266             //stop the engine,because it is in

267             // the right position

268             storeData(0, "outputs", HBRIDGE0);

269             //update the state

270             $state = 7;

271             whiteWait();

272         }

273         //loop

274         motorUp();

275     }

276

277     //state 7

278     void whiteWait() {

279         timerManage();

280         //we are waiting for the white disk to

281         // be sorted

282         if ($sleep == SORT * 1000) {

283             //start moving the sorter down

284             storeData(9, "outputs", HBRIDGE1);

285             //update the state

286             $state = 8;

287             //reset sleep for the next function

288             $sleep = 0;

289             motorDown();

290

291         }

292         //check if we need to pause

293         $startStop = getButtonPressed(0);

294         if ($startStop == 1) {

295             //stop the feeder engine

296             storeData(0, "outputs", FEEDERENGINE);

297             //set the timer

298             setCountdown(BELT);

299             //update the state

300             $state = 11;

301             whiteWaitTimer();

302         }

303         //loop

304         $sleep++;

305         whiteWait();

306     }

307

308     //state 8

309     void motorDown() {

310         timerManage();

311         //the sorter is moving down

312         if ($sleep == TIMEMOTORDOWN * 1000) {

313             //stop the sorter

314             storeData(0, "outputs", HBRIDGE1);

315             //update the state

316             $state = 9;

317             //reset sleep for the next function

318             $sleep = 0;

319             runningWait();

320         }

321         //check if we need to pause

322         $startStop = getButtonPressed(0);

323         if ($startStop == 1) {

324             //stop the feeder engine

325             storeData(0, "outputs", FEEDERENGINE);

326             //set the timer

327             setCountdown(BELT);

328             motorDownTimer();

329         }

330         //loop

331         $sleep++;

332         motorDown();

333

334     }

335

336     //state 9

337     void runningTimer() {

338         timerManage();

339         //update state

340         $state = 13;

341         runningStop();

342     }

343

344     //state 10

345     void motorUpTimer() {

346         timerManage();

347         //update state

348         $state = 14;

349         motorUpStop();

350     }

351

352     //state 11

353     void whiteWaitTimer() {

354         timerManage();

355         //update state

356         $state = 15;

357         whiteWaitStop();

358     }

359

360     //state 12

361     void motorDownTimer() {

362         timerManage();

363         //update state

364         $state = 16;

365         motorDownStop();

366     }

367

368     //state 13

369     void runningStop() {

370         timerManage();

371         //check if a white disk is at the

372         // colour detector

373         $colour = getButtonPressed(6);

374         if ($colour == 1) {

375             //move the sorter engine up

376             storeData(9, "outputs", HBRIDGE0);

377             //update the state

378             $state = 10;

379             motorUpStop();

380         }

381         //loop

382         runningStop();

383     }

384

385     //state 14

386     void motorUpStop() {

387         timerManage();

388         //check if the sorter push button is

389         // pressed

390         $push = getButtonPressed(5);

391         if ($push == 1) {

392             //stop the engien for the sorter

393             storeData(0, "outputs", HBRIDGE0);

394             //update the state

395             $state = 11;

396             whiteWaitStop();

397         }

398         motorUpStop();

399     }

400

401     //state 15

402     void whiteWaitStop() {

403         timerManage();

404         //check if the white disk has been sorted

405         if ($sleep == SORT * 1000) {

406             //start moving the sorter down

407             storeData(9, "outputs", HBRIDGE1);

408             //update the state

409             $state = 12;

410             //reset the sleep for the next

411             // function

412             $sleep = 0;

413             motorDown();

414         }

415         //loop

416         $sleep++;

417         whiteWaitStop();

418     }

419

420     //state 16

421     void motorDownStop() {

422         timerManage();

423         //check if the sorter has moved down

424         if ($sleep == TIMEMOTORDOWN) {

425             //stop the engine of the sorter

426             storeData(0, "outputs", HBRIDGE1);

427             //update the state

428             $state = 9;

429             //reset sleep for the next function

430             $sleep = 0;

431             runningWait();

432         }

433         //loop

434         $sleep++;

435         motorDownStop();

436     }

437

438     //not a state

439     void timerInterrupt() {

440         //show that we have timer interrupt

441         $state = 18;

442         //make the sorter move up

443         storeData(9, "outputs", HBRIDGE0);

444         //stop all other outputs

445         storeData(0, "outputs", HBRIDGE1);

446         storeData(0, "outputs", LENSLAMPPOSITION);

447         storeData(0, "outputs", LENSLAMPSORTER);

448         storeData(0, "outputs",

449                   LEDSTATEINDICATOR);

450         storeData(0, "outputs", DISPLAY);

451         storeData(0, "outputs", CONVEYORBELT);

452         storeData(0, "outputs", FEEDERENGINE);

453         //make sure that the outputs get set

454         // immediatly

455         timerManage();

456         //set the display to the state of initial

457         $state = 0;

458

459         initial();

460

461     }

462

463     void abort() {

464         //stop all outputs

465         storeData(0, "outputs", HBRIDGE0);

466         storeData(0, "outputs", HBRIDGE1);

467         storeData(0, "outputs", LENSLAMPPOSITION);

468         storeData(0, "outputs", LENSLAMPSORTER);

469         storeData(0, "outputs",

470                   LEDSTATEINDICATOR);

471         storeData(0, "outputs", DISPLAY);

472         storeData(0, "outputs", CONVEYORBELT);

473         storeData(0, "outputs", FEEDERENGINE);

474         //make sure the outputs stop immediatly

475         timerManage();

476         //update the state to be correct in

477         // aborted

478         $state = 17;

479         aborted();

480

481     }

482

483     //state 17

484     void aborted() {

485         timerManage();

486         //check if we can start again

487         $startStop = getButtonPressed(0);

488         if ($startStop == 1) {

489             //start moving the sorter up for

490             // calibration

491             storeData(1, "outputs", HBRIDGE0);

492             //update the state

493             $state = 0;

494             initial();

495         }

496         //loop

497         aborted();

498

499     }

500

501     void timerManage() {

502

503

504         //make sure that when counter can not

505         // be higher than 12

506         mod(13, $counter);

507         //get the voltage of output $location

508         int $voltage = getData("outputs",

509                                $location);

510         //power up the output when it needs to

511         if ($voltage > $counter) {

512             $engines += pow(2, $voltage);

513         }

514         //check if we are in a new itteration

515         if ($counter == 0) {

516             //set the first part of the display

517             $temp = getData("state", 0);

518             mod(10, $temp);

519             display($temp, "display", "1");

520

521

522         }

523         //check if we are at the end of the

524         // itteration

525         if ($counter == 12) {

526             //set the second part of the display;

527             $temp = getData("state", 0);

528             $temp = $temp / 10;

529             mod(10, $temp);

530             display($temp, "display", "01");

531

532         }

533         //check if we did all outputs

534         if ($location > 7) {

535             display($engines, "leds", "");

536             //set the variables for the next run

537             $engines = 0;

538             $location = 0;

539             $counter++;

540

541             //check if abort is pressed

542             $abort = getButtonPressed(1);

543             if ($abort == 1) {

544                 abort();//stop the machine

545             }

546             return;

547         }

548

549

550         $location++;

551         timerManage();

552     }

553 }

# Appendix 3: Explanation of the compiler

The compiler works in phases. We will go through these phases 1 by 1 to explain how the compiler does its job: compiling PHP-like code to assembly. Throughout the phases the compiler keeps track of the line number of the PHP code it is currently compiling and uses that, when an error occurs, to give information where the error is. The compiler is written in PHP5.6 and uses a command line interface.

Preprocessing

In the first phase, the input code will be made ready for the next steps. A few things happen in this phase: First the file is read into the memory. The next step is that all comments, newlines and extra spaces are stripped from the file. The file is then split into single lines using the “;” symbol that denotes the end of a line. The code is divided in three segments. The first segment starts at //**COMPILER, everything before this statement is ignored.

The preprocessor further removes some special statements that are needed to make valid php such as “global” and changes some shortcuts in their full version. For example $abc++ will be changed into $abc+=1. This ensures that the compiler only needs to be able to handle $abc+=1.

Splitting

In the second phase the code is split up by function. Every function gets his own array with all the lines that are in that function. The code not inside of a function goes into a separate array.

Compiling

The third phase is the most important one. It starts by compiling the code that is at the start and not inside a function. While compiling it keeps track of what functions are called and adds these, if they are not already compiled, to the toCompile queue. This helps in making sure there is no dead code, as a function that is never called, will not be compiled. The compiler adds the function “main”, which is the default start point of the code, to the queue and starts processing it.

After compiling the main function it will continue in the next function in the toCompile queue and keep doing this till the toCompile queue is empty.

The compiling itself is not a lot more than a lot of regex and switch statements that look at the input and make a output from that. At the first notion of a variable a register is assigned to it. The code then uses this register in place of the variable. Some more difficult statements, like the function display which displays something, will BRS to premade assembly code that handles that. The compiler keeps track of which segments of the premade assembly code are used.

When the compiler meets an if statement, it saves the code inside it to a new function named “condtionali” where i is the amount of conditionals that have already been seen. It then places this function in the toCompile queue. It also saves the location of the end of the if statement, so it will later know where to return when the if function has ended.

For every line it compiles, it takes the corresponding line of PHP and inserts it as a comment in the assembly. This is to help in debugging.

Combining

After there are no functions left in the toCompile queue, the combining phase starts. In this phase all the functions and the code outside the functions are combined into a single array. This phase also adds the used premade functions at the top and inserts the return statements at the correct position.

Formatting

The last phase is the least interesting. It goes through the, now compiled code, and formats it. It uses either the length of the longest function name or the number 25 depending on which is larger to insert spaces in front of every line of code in a way everything lines up nicely. It also makes sure the comments line up nicely.

The last step the compiler takes is writing the compiled code to a file and using the assembler provided to create the hex code.

# Appendix 4: Explanation of the compiler functions

storeRam($location, $value)

Store a value in the ram.

$location		The location (a variable) to store the value in the ram

$value			The value to store, needs to be a variable

return			void

getRam($location)

Get a value from the ram.

$location		The location (a variable) where the value is stored

return			The value that is stored at the location

display($what, $onWhat, $location = '000001')

Display something on either the display or the leds.

Possible values for $onwhat:

- leds: the leds at the top

- leds2: the leds to the right

- display: the display

$what			What to display, must be a variable

$onWhat		On what to display

$location		Where to show the value when using the display, defaults to the right position

return			void

pow($number,$power)

Get the power of a number

$number		The number to power

$power		The power value

return			Int; The result

mod($what, $variable)

Take the modulo of a number

$what			Modulo what

$variable		Variable to modulo over

return		void

getInput($writeTo, $type)

Get button or analog input. When you just want the input of 1 button, use getButtonPressed instead.

$writeTo		Variable to write the input to

$type			Type of input, possible values are: buttons, analog

return			void

getButtonPressed($button)

Check if a button is pressed. Puts the result into R5.

$button		Which button to check (input a variable)

return			Int; Whether or not the button is pressed.

installCountdown($functionName)

Install the countdown.

$functionName	The name of the function where the timer should go to

return			void

startCountdown()

Start the countdown.

Retrun			void

pushStack($variable)

Push a variable to the stack

$variable		The variable to push to the stack

return			void

pullStack($variable)

Pull a variable from the stack.

$variable		The variable where the pulled variable is put into

return			void

setCountdown($countdown)

Set the timer interrupt to a value. It will first reset the timer to 0.

$countdown		How long the countdown should wait, in timer ticks

return			void

getData($location, $offset)

Get data. Use offset 0 when it is just a single value.

$location		The location where the variable is stored

$offset			The offset of the location

return			The value of the data segment

storeData($variable, $location, $offset)

Store data. Use offset 0 when it is just a single value.

$variable		The variable to store

$location		The name of the location where the variable is stored

$offset			The offset of the location

return			void

sleep($howLong)

Pause the program.

$howLong		How long to sleep in clockticks

return			void

initVar($variable,$places)

Initialize a variable that is used in that data segment.

$variable		The name of the variable

$places		How long the array is

return			void

branch($branchTO)

Branch to a function.

$branchTO		where to branch to

return			void

moveFunction($branchTO)

Move a function in the assembly code.

$branchTO		Where to branch to

return			void

Appendix 5: PHP Program

1 <?php

2 /* vim: set expandtab tabstop=4 shiftwidth=4 softtabstop=4: */

3

4 /**

5  * Sort of a simulation of the PP2 program controlling the Fischer Technik in order to sort black and white discs.

6  * @team Group 16

7  * @author Stefan van den Berg

8  * @author Rolf Verschuuren

9  * @author Wigger Boelens

10  * @since 13/3/2015

11  */

12 include 'functions.php';

13 //**COMPILER**

14 moveFunction('timerInterrupt', 1);

15 moveFunction('timerManage', 50);

16

17 //**DATA**

18 initVar('offset', 1);

19 initVar('stackPointer', 1);

20 initVar('outputs', 12);

21 initVar('state', 1);

22

23 //**CODE**

24 define('TIMEMOTORDOWN', 150); //how long the sorter takes to move down

25 define('BELT', 2000);

26 define('BELTROUND', 2000);//Time for the belt to make a rotation

27 define('SORT', 200);//Clockticks to make a rotation

28 define('COUNTDOWN', 30000);

29 //outputs

30 define('LENSLAMPPOSITION', 2);

31 define('LENSLAMPSORTER', 6);

32 define('HBRIDGE0', 0);

33 define('HBRIDGE1', 1);

34 define('CONVEYORBELT', 7);

35 define('FEEDERENGINE', 3);

36 define('DISPLAY', 8);

37 define('LEDSTATEINDICATOR', 9);

38

39 //not a state

40 function main()

41 {

42     global $counter, $location;

43

44     //store the offset of the program, this is used in the interrupt

45     storeData(R5, 'offset', 0);

46     //install the countdown

47     installCountdown('timerInterrupt');

48

49     //save the location of the stackPointer, so we can clear the stack

50     storeData(SP, 'stackPointer', 0);

51

52     //the variables that are the same throughout the program:

53     $counter = 0;

54     $location = 0;

55     $sleep = 0;

56

57

58     //stop everything

59     $temp = 0;

60     storeData($temp, 'outputs', HBRIDGE1);

61     storeData($temp, 'outputs', LENSLAMPPOSITION);

62     storeData($temp, 'outputs', LENSLAMPSORTER);

63     storeData($temp, 'outputs', LEDSTATEINDICATOR);

64     storeData($temp, 'outputs', DISPLAY);

65     storeData($temp, 'outputs', CONVEYORBELT);

66     storeData($temp, 'outputs', FEEDERENGINE);

67

68     //sh0w the state

69     $state = 0;

70     storeData($state, 'state', 0);

71

72     //set HBridge so the sorter starts moving up

73     $temp = 10;

74     storeData($temp, 'outputs', HBRIDGE0);

75     unset($temp, $state);

76

77     //go to the first state

78     initial();

79 }

80

81 //state 0

82 function initial()

83 {

84     global $sleep;

85     //disable the lights on the right hand side

86     $temp = 0;

87     display($temp, 'leds2');

88

89     $temp = getData('stackPointer', 0);

90     setStackPointer($temp);

91

92     timerManage();

93

94     //check if the sorter push button is pressed

95     $push = getButtonPressed(5);

96     if ($push == 1) {

97         //move sorter down

98         $temp = 0;

99         storeData($temp, 'outputs', HBRIDGE0);

100         $temp = 10;

101         storeData($temp, 'outputs', HBRIDGE1);

102

103         //update state

104         $temp = 1;

105         storeData($temp, 'state', 0);

106         unset($temp);

107

108         //reset sleep for the next function

109         $sleep = 0;

110         calibrateSorter();

111

112     }

113     unset($push);

114

115     //loop

116     initial();

117 }

118

119 //state 1

120 function calibrateSorter()

121 {

122     global $sleep;

123     timerManage();

124

125     //the sorter is now moving down,

126     //we're waiting for it to reach its bottom position

127     if ($sleep == TIMEMOTORDOWN) {

128         //stop the sorter

129         $temp = 0;

130         storeData($temp, 'outputs', HBRIDGE1);

131

132         //update the state

133         $state = 2;

134         storeData($state, 'state', 0);

135         unset($state);

136

137         //reset sleep for the next state

138         $sleep = 0;

139         resting();

140     }

141

142     //loop

143     $sleep++;

144     calibrateSorter();

145 }

146

147 //state 2

148 function resting()

149 {

150     timerManage();

151

152     //the program is now waiting for the user to press start/stop

153     $startStop = getButtonPressed(0);

154     if ($startStop == 1) {

155         //sleep so we don't go to pause immediately

156

157

158         //power up the lamps

159         $temp = 12;

160         storeData($temp, 'outputs', LENSLAMPPOSITION);

161         unset($temp);

162         timerManage();

163         sleep(1000);

164         $temp = 12;

165         storeData($temp, 'outputs', LENSLAMPSORTER);

166         unset($temp);

167         timerManage();

168         sleep(2000);

169

170

171         //start up the belt and feeder

172         $temp = 9;

173         storeData($temp, 'outputs', CONVEYORBELT);

174         $temp = 9;

175         storeData($temp, 'outputs', FEEDERENGINE);

176         unset($temp);

177

178         //set and start the countdown for the moment there are no more disks

179         //this countdown will reset every time a disk is found

180         //when it triggers, timerInterrupt will be ran.

181         setCountdown(COUNTDOWN);

182         startCountdown();

183

184         //update the state

185         $state = 3;

186         storeData($state, 'state', 0);

187         unset($state);

188

189         running();

190     }

191     unset($startStop);

192

193     //loop

194     resting();

195 }

196

197 //state 3

198 function running()

199 {

200     timerManage();

201

202     //check if we need to pause

203     $startStop = getButtonPressed(0);

204     if ($startStop == 1) {

205         //stop the feeder engine

206         $temp = 0;

207         storeData($temp, 'outputs', FEEDERENGINE);

208         unset($temp);

209

210         //exit after 1 rotation of the belt

211         setCountdown(BELT * 10);

212

213         //update the state

214         $state = 9;//TODO: echte state

215         storeData($state, 'state', 0);

216         unset($state);

217

218         runningTimer();

219

220     }

221     unset($startStop);

222

223     //check if a disk is at the position detector

224     $position = getButtonPressed(7);

225     if ($position == 0) {

226         //reset the countdown, because a disk was just detected

227         setCountdown(COUNTDOWN);

228

229         //update the state

230         $state = 4;

231         storeData($state, 'state', 0);

232         unset($state);

233         runningWait();

234     }

235     unset($position);

236

237     //loop

238     running();

239 }

240

241 //state 4

242 function runningWait()

243 {

244     timerManage();

245

246     //check if we need to pause

247     $startStop = getButtonPressed(0);

248     if ($startStop == 1) {

249         //stop the feeder engine

250         $temp = 0;

251         storeData($temp, 'outputs', FEEDERENGINE);

252         unset($temp);

253

254         //exit after 1 rotation of the belt

255         setCountdown(BELT * 10);

256

257         //update the state

258         $state = 9;

259         storeData($state, 'state', 0);

260         unset($state);

261

262         runningTimer();

263

264     }

265     unset($startStop);

266

267     //check if a disk is at the position detector

268     $position = getButtonPressed(7);

269     if ($position == 0) {

270         //reset the countdown, because a disk was just detected

271         setCountdown(COUNTDOWN);

272

273         //update state

274         $state = 5;

275         storeData($state, 'state', 0);

276         unset($state);

277

278         runningTimerReset();

279

280     }

281     unset($position);

282

283     //check if a white disk is at the colour detector

284     $colour = getButtonPressed(6);

285     if ($colour == 1) {

286         //move the sorter up so the disk goes to the correct box

287         $temp = 10;

288         storeData($temp, 'outputs', HBRIDGE0);

289

290         //stop the feeder engine

291         $temp = 0;

292         storeData($temp, 'outputs', FEEDERENGINE);

293         unset($temp);

294

295         //update state

296         $state = 6;

297         storeData($state, 'state', 0);

298         unset($state);

299

300         motorUp();

301     }

302     unset($colour);

303

304     //loop

305     runningWait();

306 }

307

308 //state 5

309 function runningTimerReset()

310 {

311     timerManage();

312

313     //update state

314     $state = 4;

315     storeData($state, 'state', 0);

316     unset($state);

317

318     runningWait();

319 }

320

321 //state 6

322 function motorUp()

323 {

324     global $sleep;

325     timerManage();

326

327     //check if we need to pause

328     $startStop = getButtonPressed(0);

329     if ($startStop == 1) {

330         //stop the feeder engine

331         $temp = 0;

332         storeData($temp, 'outputs', FEEDERENGINE);

333         unset($temp);

334

335         //exit after 1 rotation of the belt

336         setCountdown(BELT * 10);

337

338         //update the state

339         $state = 10;

340         storeData($state, 'state', 0);

341         unset($state);

342

343         motorUpTimer();

344

345     }

346     unset($startStop);

347

348     //check if the sorter push button is pressed

349     $push = getButtonPressed(5);

350     if ($push == 1) {

351         //stop the sorter engine, because its at its highest position

352         $temp = 0;

353         storeData($temp, 'outputs', HBRIDGE0);

354         unset($temp);

355

356         //update state

357         $state = 7;

358         storeData($state, 'state', 0);

359         unset($state);

360

361         //set sleep for the next function

362         $sleep = 0;

363

364         whiteWait();

365     }

366     unset($push);

367

368     //loop

369     motorUp();

370 }

371

372 //state 7

373 function whiteWait()

374 {

375     global $sleep;

376     timerManage();

377

378     //we are waiting for the white disk to be sorted

379     if ($sleep == SORT) {

380         //start moving the sorter down

381         $temp = 10;

382         storeData($temp, 'outputs', HBRIDGE1);

383         unset($temp);

384

385         //make sure the timerinterrupt is correct

386         setCountdown(COUNTDOWN);

387

388         //update state

389         $state = 8;

390         storeData($state, 'state', 0);

391         unset($state);

392

393         //reset sleep for the next function

394         $sleep = 0;

395         motorDown();

396

397     }

398

399     //check if we need to pause

400     $startStop = getButtonPressed(0);

401     if ($startStop == 1) {

402         //stop the feeder engine

403         $temp = 0;

404         storeData($temp, 'outputs', FEEDERENGINE);

405         unset($temp);

406

407         //exit after 1 rotation of the belt

408         setCountdown(BELT * 10);

409

410         //update the state

411         $state = 11;

412         storeData($state, 'state', 0);

413         unset($state);

414

415         whiteWaitTimer();

416     }

417     unset($startStop);

418

419     //loop

420     $sleep++;

421     whiteWait();

422 }

423

424 //state 8

425 function motorDown()

426 {

427     global $sleep;

428     timerManage();

429

430

431     //check if a white disk is at the colour detector

432     $colour = getButtonPressed(6);

433     if ($colour == 1) {

434         //move the sorter up so the disk goes to the correct box

435         $temp=0;

436         storeData($temp,'outputs',HBRIDGE1);

437         $temp = 10;

438         storeData($temp, 'outputs', HBRIDGE0);

439         unset($temp);

440

441         //update state

442         $state = 6;

443         storeData($state, 'state', 0);

444         $sleep=0;

445         unset($state);

446

447         motorUp();

448     }

449     unset($colour);

450

451

452     //the sorter is moving down, we are waiting for that to complete

453     if ($sleep == TIMEMOTORDOWN) {

454         //stop the sorter, its where it should be

455         $temp = 0;

456         storeData($temp, 'outputs', HBRIDGE1);

457         $temp = 7;

458         storeData($temp, 'outputs', FEEDERENGINE);

459         unset($temp);

460

461         //update state

462         $state = 4;

463         storeData($state, 'state', 0);

464         //reset sleep for the next function

465         $sleep = 0;

466         unset($state);

467

468         runningWait();

469     }

470

471     //check if we need to pause

472     $startStop = getButtonPressed(0);

473     if ($startStop == 1) {

474         //stop the feeder engine

475         $temp = 0;

476         storeData($temp, 'outputs', FEEDERENGINE);

477         unset($temp);

478

479         //exit after 1 rotation of the belt

480         setCountdown(BELT * 10);

481

482         //update the state

483         $state = 12;

484         storeData($state, 'state', 0);

485         unset($state);

486

487         motorDownTimer();

488     }

489     unset($startStop);

490

491     //loop

492     $sleep++;

493     motorDown();

494

495 }

496

497 //state 9

498 function runningTimer()

499 {

500     timerManage();

501

502     //update state

503     $state = 13;

504     storeData($state, 'state', 0);

505     unset($state);

506

507     runningStop();

508 }

509

510 //state 10

511 function motorUpTimer()

512 {

513     timerManage();

514

515     //update state

516     $state = 14;

517     storeData($state, 'state', 0);

518     unset($state);

519

520     motorUpStop();

521 }

522

523 //state 11

524 function whiteWaitTimer()

525 {

526     timerManage();

527

528     //update state

529     $state = 15;

530     storeData($state, 'state', 0);

531     unset($state);

532

533     whiteWaitStop();

534 }

535

536 //state 12

537 function motorDownTimer()

538 {

539     timerManage();

540

541     //update state

542     $state = 16;

543     storeData($state, 'state', 0);

544     unset($state);

545

546     motorDownStop();

547 }

548

549 //state 13

550 function runningStop()

551 {

552     timerManage();

553

554     //check if a white disk is at the colour detector

555     $colour = getButtonPressed(6);

556     if ($colour == 1) {

557         //stop the sorter engine, because its at its highest position

558         $temp = 10;

559         storeData($temp, 'outputs', HBRIDGE0);

560

561         //stop the feeder engine

562         $temp = 0;

563         storeData($temp, 'outputs', FEEDERENGINE);

564         unset($temp);

565

566         //update state

567         $state = 10;

568         storeData($state, 'state', 0);

569         unset($state);

570

571         motorUpStop();

572     }

573     unset($colour);

574

575     //loop

576     runningStop();

577 }

578

579 //state 14

580 function motorUpStop()

581 {

582     timerManage();

583

584     //check if the sorter push button is pressed

585     $push = getButtonPressed(5);

586     if ($push == 1) {

587         //stop the engine of the sorter

588         $temp = 0;

589         storeData($temp, 'outputs', HBRIDGE0);

590         unset($temp);

591

592         //update state

593         $state = 11;

594         storeData($state, 'state', 0);

595         unset($state);

596

597         whiteWaitStop();

598     }

599     unset($push);

600

601     //loop

602     motorUpStop();

603 }

604

605 //state 15

606 function whiteWaitStop()

607 {

608     global $sleep;

609     timerManage();

610

611     //check if the white disk has been sorted

612     if ($sleep == SORT) {

613         //it has, so lets start moving the sorter down

614         $temp = 10;

615         storeData($temp, 'outputs', HBRIDGE1);

616         $temp = 0;

617         storeData($temp, 'outputs', FEEDERENGINE);

618         unset($temp);

619

620         //update state

621         $state = 12;

622         storeData($state, 'state', 0);

623         unset($state);

624

625         $sleep = 0;

626         motorDownStop();

627     }

628

629     //loop

630     $sleep++;

631     whiteWaitStop();

632 }

633

634 //state 16

635 function motorDownStop()

636 {

637     global $sleep;

638     timerManage();

639

640     //check if the sorter has moved down

641     if ($sleep == TIMEMOTORDOWN) {

642         //it has, so lets stop it

643         $temp = 0;

644         storeData($temp, 'outputs', HBRIDGE1);

645         unset($temp);

646

647         //update the state

648         $state = 9;

649         storeData($state, 'state', 0);

650         unset($state);

651

652         $sleep = 0;

653         runningStop();

654     }

655

656     //loop

657     $sleep++;

658     motorDownStop();

659 }

660

661 //not a state

662 function timerInterrupt()

663 {

664     timerManage();

665     //show that we are in the timer interrupt

666     $temp = 5;

667     display($temp, 'display');

668

669     //start moving the sorter up, to start the calibration

670     $temp = 10;

671     storeData($temp, 'outputs', HBRIDGE0);

672

673     //stop the rest

674     $temp = 0;

675     storeData($temp, 'outputs', LENSLAMPPOSITION);

676     storeData($temp, 'outputs', LENSLAMPSORTER);

677     storeData($temp, 'outputs', LEDSTATEINDICATOR);

678     storeData($temp, 'outputs', DISPLAY);

679     storeData($temp, 'outputs', CONVEYORBELT);

680     storeData($temp, 'outputs', FEEDERENGINE);

681

682

683     //reset, because we will no longer be in timerInterrupt

684     display($temp, 'display');

685     unset($temp);

686

687     //go back to initial

688     $temp = getData('offset', 0);

689     $temp2 = getFuncLocation('initial');

690     $temp += $temp2;

691

692

693     addStackPointer(2);

694     pushStack($temp);

695     addStackPointer(-1);

696 }

697

698 //not a state

699 function abort()

700 {

701     //free some memory

702     unset($engines);

703

704     //prevent timerinterrupt

705     setCountdown(1000);

706     $temp = getData('stackPointer', 0);

707     setStackPointer($temp);

708

709     //stop everything

710     $temp = 0;

711     storeData($temp, 'outputs', HBRIDGE1);

712     storeData($temp, 'outputs', HBRIDGE0);

713     storeData($temp, 'outputs', LENSLAMPPOSITION);

714     storeData($temp, 'outputs', LENSLAMPSORTER);

715     storeData($temp, 'outputs', LEDSTATEINDICATOR);

716     storeData($temp, 'outputs', DISPLAY);

717     storeData($temp, 'outputs', CONVEYORBELT);

718     storeData($temp, 'outputs', FEEDERENGINE);

719     unset($temp);

720

721     //apply the changes to actually stop it

722     timerManage();

723

724     //update the state

725     $state = 17;

726     storeData($state, 'state', 0);

727

728

729     //show we aborted

730     $state = 7;

731     display($state, 'leds2', 0);

732     unset($state);

733

734     aborted();

735 }

736

737 //state 17

738 function aborted()

739 {

740     //prevent timer interrupt

741     setCountdown(1000);

742     timerManage();

743     //check if we can start again

744     $startStop = getButtonPressed(0);

745     if ($startStop == 1) {

746         //start moving the sorter up, to start the calibration

747         $temp = 10;

748         storeData($temp, 'outputs', HBRIDGE0);

749         unset($temp);

750

751         //update the state

752         $state = 0;

753         storeData($state, 'state', 0);

754         unset($state);

755

756         initial();

757     }

758     unset($startStop);

759     aborted();

760

761 }

762

763 //not a state

764 function timerManage()

765 {

766     global $location, $counter, $engine, $sleep;

767

768     if ($location == 0) {

769         $engines = 0;

770     }

771

772     //makes sure that when $counter >12 it will reset to 0

773     mod(12, $counter);

774

775     //get the voltage of output $location

776     $voltage = getData('outputs', $location);

777

778     //power up the output when it needs to

779     if ($voltage > $counter) {

780         $voltage = $location;

781         $voltage = pow(2, $voltage);

782         $engines += $voltage;

783     }

784

785     //check if we did all outputs

786     if ($location == 7) {

787         //actually output the result

788         sleep(1);

789         display($engines, 'leds');

790

791

792         unset($voltage);

793         //check if abort is pressed

794         $abort = getButtonPressed(1);

795         if ($abort == 1) {

796             abort();//STOP THE MACHINE!

797         }

798         unset($abort);

799

800         //check if we are in a new iteration

801         if ($counter == 6) {

802             //set the first part of the display

803             $temp = getData('state', 0);

804             mod(10, $temp);

805             display($temp, 'display', 1);

806             unset($temp);

807         }

808         //check if we are at the end of the iteration

809         if ($counter == 11) {

810             //set the second part of the display;

811             pushStack($sleep);

812

813             $temp = getData('state', 0);

814             //get the last digit of the state

815             //we have no variables left, so we use $sleep

816

817             $sleep = $temp;

818             mod(10, $sleep);

819             $temp -= $sleep;

820             $temp /= 10;

821             //display the last digit

822             display($temp, 'display', 2);

823

824             pullStack($sleep);

825             unset($temp);

826         }

827

828

829         //set the variables for the next run

830         $engines = 0;

831         $location = 0;

832         $counter++;

833

834         //and return to where we came from

835         return;

836     }

837

838     //loop

839     $location++;

840     branch('timerManage');

841 }

# Appendix 6: Assembly Program

1 @DATA

2 offset DS 1

3 stackPointer DS 1

4 outputs DS 12

5 state DS 1

6

7 @CODE

8

9                      TIMEMOTORDOWN EQU 150

10                      BELT EQU 2000

11                      BELTROUND EQU 2000

12                      SORT EQU 200

13                      COUNTDOWN EQU 30000

14                      LENSLAMPPOSITION EQU 2

15                      LENSLAMPSORTER EQU 6

16                      HBRIDGE0 EQU 0

17                      HBRIDGE1 EQU 1

18                      CONVEYORBELT EQU 7

19                      FEEDERENGINE EQU 3

20                      DISPLAY EQU 8

21                      LEDSTATEINDICATOR EQU 9

22 begin:               BRA main

23

24

25                                                                   ;sleep

26 _timer:              MULS R5 10

27                      PUSH R4

28                      LOAD R4 R5

29                      LOAD R5 -16

30                      LOAD R5 [R5+13]

31                      SUB  R5 R4

32                      LOAD R4 -16

33 _wait:               CMP  R5 [R4+13]                              ; Compare the timer to 0

34                      BMI  _wait

35                      PULL R4

36                      RTS

37

38 _pressed:            PUSH R4                                      ;make sure all vars are the same at the end

39                      PUSH R5

40                      LOAD R4 R3

41                      LOAD R5 2

42                      BRS _pow

43                      LOAD R3 R5

44                      LOAD R5 -16

45                      LOAD R4 [R5+7]

46                      DIV R4 R3

47                      MOD R4 2

48

49                      PUSH R4                                      ;the result

50                      ADD SP 1                                     ;decrease the SP so we get the correct pulls

51

52                      PULL R5

53                      PULL R4

54

55                      RTS

56

57 _pow:                CMP R4 0

58                      BEQ _pow1

59                      CMP R4 1

60                      BEQ _powR

61                      PUSH R3

62                      PUSH R4

63                      SUB R4 1

64                      LOAD R3 R5

65 _powLoop:            MULS R5 R3

66                      SUB R4 1

67                      CMP R4 0

68                      BEQ _powReturn

69                      BRA _powLoop

70 _powReturn:          PULL R4

71                      PULL R3

72                      RTS

73 _pow1:               LOAD R5 1

74                      RTS

75 _powR:               RTS

76

77                                                                   ;display

78 _Hex7Seg:            BRS _Hex7Seg_bgn                             ; push address(tbl) onto stack and proceed at bgn

79 _Hex7Seg_tbl:        CONS %01111110                               ; 7-segment pattern for '0'

80                      CONS %00110000                               ; 7-segment pattern for '1'

81                      CONS %01101101                               ; 7-segment pattern for '2'

82                      CONS %01111001                               ; 7-segment pattern for '3'

83                      CONS %00110011                               ; 7-segment pattern for '4'

84                      CONS %01011011                               ; 7-segment pattern for '5'

85                      CONS %01011111                               ; 7-segment pattern for '6'

86                      CONS %01110000                               ; 7-segment pattern for '7'

87                      CONS %01111111                               ; 7-segment pattern for '8'

88                      CONS %01111011                               ; 7-segment pattern for '9'

89                      CONS %01110111                               ; 7-segment pattern for 'A'

90                      CONS %00011111                               ; 7-segment pattern for 'b'

91                      CONS %01001110                               ; 7-segment pattern for 'C'

92                      CONS %00111101                               ; 7-segment pattern for 'd'

93                      CONS %01001111                               ; 7-segment pattern for 'E'

94                      CONS %01000111                               ; 7-segment pattern for 'F'

95 _Hex7Seg_bgn:        AND R5 %01111                                ; R0 = R0 MOD 16 , just to be safe...

96                      LOAD R4 [SP++]                               ; R4 = address(tbl) (retrieve from stack)

97                      LOAD R4 [R4+R5]                              ; R4 = tbl[R0]

98                      LOAD R5 -16

99                      STOR R4 [R5+8]                               ; and place this in the Display Element

100                      RTS

101 main:                STOR R5 [GB +offset + 0]                     ;storeData(R5, 'offset', 0)

102                      LOAD R0 timerInterrupt                       ;installCountdown('timerInterrupt')

103                      ADD R0 R5

104                      LOAD R1 16

105                      STOR R0 [R1]

106

107                      LOAD R5 -16

108

109                                                                   ; Set the timer to 0

110                      LOAD R0 0

111                      SUB R0 [R5+13]

112                      STOR R0 [R5+13]

113                      STOR SP [GB +stackPointer + 0]               ;storeData(SP, 'stackPointer', 0)

114                      LOAD R0 0                                    ;$counter = 0

115                      LOAD R1 0                                    ;$location = 0

116                      LOAD R2 0                                    ;$sleep = 0

117                      LOAD R3 0                                    ;$temp = 0

118                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

119                      STOR R3 [GB +outputs + LENSLAMPPOSITION]     ;storeData($temp, 'outputs', LENSLAMPPOSITION)

120                      STOR R3 [GB +outputs + LENSLAMPSORTER]       ;storeData($temp, 'outputs', LENSLAMPSORTER)

121                      STOR R3 [GB +outputs + LEDSTATEINDICATOR]    ;storeData($temp, 'outputs', LEDSTATEINDICATOR)

122                      STOR R3 [GB +outputs + DISPLAY]              ;storeData($temp, 'outputs', DISPLAY)

123                      STOR R3 [GB +outputs + CONVEYORBELT]         ;storeData($temp, 'outputs', CONVEYORBELT)

124                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

125                      LOAD R4 0                                    ;$state = 0

126                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

127                      LOAD R3 10                                   ;$temp = 10

128                      STOR R3 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

129                                                                   ;unset($temp, $state)

130                      BRA initial                                  ;initial()

131

132 timerInterrupt:      BRS timerManage                              ;timerManage()

133                      LOAD R3 5                                    ;$temp = 5

134                      PUSH R5                                      ;display($temp, 'display')

135                      PUSH R4

136                      LOAD R5 R3

137                      BRS _Hex7Seg

138                      LOAD R4 %0000001

139                      STOR R4 [R5+9]

140                      PULL R4

141                      PULL R5

142                      LOAD R3 10                                   ;$temp = 10

143                      STOR R3 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

144                      LOAD R3 0                                    ;$temp = 0

145                      STOR R3 [GB +outputs + LENSLAMPPOSITION]     ;storeData($temp, 'outputs', LENSLAMPPOSITION)

146                      STOR R3 [GB +outputs + LENSLAMPSORTER]       ;storeData($temp, 'outputs', LENSLAMPSORTER)

147                      STOR R3 [GB +outputs + LEDSTATEINDICATOR]    ;storeData($temp, 'outputs', LEDSTATEINDICATOR)

148                      STOR R3 [GB +outputs + DISPLAY]              ;storeData($temp, 'outputs', DISPLAY)

149                      STOR R3 [GB +outputs + CONVEYORBELT]         ;storeData($temp, 'outputs', CONVEYORBELT)

150                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

151                      PUSH R5                                      ;display($temp, 'display')

152                      PUSH R4

153                      LOAD R5 R3

154                      BRS _Hex7Seg

155                      LOAD R4 %0000001

156                      STOR R4 [R5+9]

157                      PULL R4

158                      PULL R5

159                                                                   ;unset($temp)

160                      LOAD R3 [ GB + offset + 0 ]                  ;$temp = getData('offset', 0)

161                      LOAD R4 initial                              ;$temp2 = getFuncLocation('initial')

162                      ADD R3 R4                                    ;$temp += $temp2

163                      ADD SP 2                                     ;addStackPointer(2)

164                      PUSH R3                                      ;pushStack($temp)

165                      ADD SP -1                                    ;addStackPointer(-1)

166                      RTE

167

168 initial:             LOAD R3 0                                    ;$temp = 0

169                      PUSH R5                                      ;display($temp, 'leds2')

170                      LOAD R5 -16

171                      STOR R3 [R5+10]

172                      PULL R5

173                      LOAD R3 [ GB + stackPointer + 0 ]            ;$temp = getData('stackPointer', 0)

174                      LOAD SP R3                                   ;setStackPointer($temp)

175                      BRS timerManage                              ;timerManage()

176                      PUSH R3                                      ;$push = getButtonPressed(5)

177                      LOAD R3 5

178                      BRS _pressed

179                      PULL R3

180                      SUB SP 5

181                      PULL R4

182                      ADD SP 4

183                      CMP R4 1                                     ;if ($push == 1) {

184                      BEQ conditional0

185 return0:                                                          ;unset($push)

186                      BRA initial                                  ;initial()

187

188                                                                   ;if ($push == 1) {

189 conditional0:        LOAD R3 0                                    ;$temp = 0

190                      STOR R3 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

191                      LOAD R3 10                                   ;$temp = 10

192                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

193                      LOAD R3 1                                    ;$temp = 1

194                      STOR R3 [GB +state + 0]                      ;storeData($temp, 'state', 0)

195                                                                   ;unset($temp)

196                      LOAD R2 0                                    ;$sleep = 0

197                      BRA calibrateSorter                          ;calibrateSorter()

198

199 calibrateSorter:     BRS timerManage                              ;timerManage()

200                      CMP R2 TIMEMOTORDOWN                         ;if ($sleep == TIMEMOTORDOWN) {

201                      BEQ conditional1

202 return1:             ADD R2 1                                     ;$sleep+=1

203                      BRA calibrateSorter                          ;calibrateSorter()

204

205                                                                   ;if ($sleep == TIMEMOTORDOWN) {

206 conditional1:        LOAD R3 0                                    ;$temp = 0

207                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

208                      LOAD R4 2                                    ;$state = 2

209                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

210                                                                   ;unset($state)

211                      LOAD R2 0                                    ;$sleep = 0

212                      BRA resting                                  ;resting()

213

214 resting:             BRS timerManage                              ;timerManage()

215                      PUSH R3                                      ;$startStop = getButtonPressed(0)

216                      LOAD R3 0

217                      BRS _pressed

218                      PULL R3

219                      SUB SP 5

220                      PULL R4

221                      ADD SP 4

222                      CMP R4 1                                     ;if ($startStop == 1) {

223                      BEQ conditional2

224 return2:                                                          ;unset($startStop)

225                      BRA resting                                  ;resting()

226

227                                                                   ;if ($startStop == 1) {

228 conditional2:        LOAD R3 12                                   ;$temp = 12

229                      STOR R3 [GB +outputs + LENSLAMPPOSITION]     ;storeData($temp, 'outputs', LENSLAMPPOSITION)

230                                                                   ;unset($temp)

231                      BRS timerManage                              ;timerManage()

232                      PUSH R5                                      ;sleep(1000)

233                      LOAD R5 1000

234                      BRS _timer

235                      PULL R5

236                      LOAD R3 12                                   ;$temp = 12

237                      STOR R3 [GB +outputs + LENSLAMPSORTER]       ;storeData($temp, 'outputs', LENSLAMPSORTER)

238                                                                   ;unset($temp)

239                      BRS timerManage                              ;timerManage()

240                      PUSH R5                                      ;sleep(2000)

241                      LOAD R5 2000

242                      BRS _timer

243                      PULL R5

244                      LOAD R3 9                                    ;$temp = 9

245                      STOR R3 [GB +outputs + CONVEYORBELT]         ;storeData($temp, 'outputs', CONVEYORBELT)

246                      LOAD R3 9                                    ;$temp = 9

247                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

248                                                                   ;unset($temp)

249                      PUSH R5 ;reset timer                         ;setCountdown(COUNTDOWN)

250                      PUSH R4

251                      LOAD R5 -16

252                      LOAD R4 0

253                      SUB R4 [R5+13]

254                      STOR R4 [R5+13]                              ;set timer

255                      LOAD R4 COUNTDOWN

256                      STOR R4 [R5+13]

257                      PULL R4

258                      PULL R5

259                      SETI 8                                       ;startCountdown()

260                      LOAD R3 3                                    ;$state = 3

261                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

262                                                                   ;unset($state)

263                      BRA running                                  ;running()

264

265 running:             BRS timerManage                              ;timerManage()

266                      PUSH R3                                      ;$startStop = getButtonPressed(0)

267                      LOAD R3 0

268                      BRS _pressed

269                      PULL R3

270                      SUB SP 5

271                      PULL R3

272                      ADD SP 4

273                      CMP R3 1                                     ;if ($startStop == 1) {

274                      BEQ conditional3

275 return3:                                                          ;unset($startStop)

276                      PUSH R3                                      ;$position = getButtonPressed(7)

277                      LOAD R3 7

278                      BRS _pressed

279                      PULL R3

280                      SUB SP 5

281                      PULL R3

282                      ADD SP 4

283                      CMP R3 1                                     ;if ($position == 1) {

284                      BEQ conditional4

285 return4:                                                          ;unset($position)

286                      BRA running                                  ;running()

287

288                                                                   ;if ($startStop == 1) {

289 conditional3:        LOAD R4 0                                    ;$temp = 0

290                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

291                                                                   ;unset($temp)

292                      PUSH R5 ;reset timer                         ;setCountdown(BELT * 10)

293                      PUSH R4

294                      LOAD R5 -16

295                      LOAD R4 0

296                      SUB R4 [R5+13]

297                      STOR R4 [R5+13]                              ;set timer

298                      LOAD R4 BELT * 10

299                      STOR R4 [R5+13]

300                      PULL R4

301                      PULL R5

302                      LOAD R4 9                                    ;$state = 9

303                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

304                                                                   ;unset($state)

305                      BRA runningTimer                             ;runningTimer()

306

307                                                                   ;if ($position == 1) {

308 conditional4:        PUSH R5 ;reset timer                         ;setCountdown(COUNTDOWN)

309                      PUSH R4

310                      LOAD R5 -16

311                      LOAD R4 0

312                      SUB R4 [R5+13]

313                      STOR R4 [R5+13]                              ;set timer

314                      LOAD R4 COUNTDOWN

315                      STOR R4 [R5+13]

316                      PULL R4

317                      PULL R5

318                      LOAD R4 4                                    ;$state = 4

319                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

320                                                                   ;unset($state)

321                      BRA runningWait                              ;runningWait()

322

323 runningWait:         BRS timerManage                              ;timerManage()

324                      PUSH R3                                      ;$startStop = getButtonPressed(0)

325                      LOAD R3 0

326                      BRS _pressed

327                      PULL R3

328                      SUB SP 5

329                      PULL R3

330                      ADD SP 4

331                      CMP R3 1                                     ;if ($startStop == 1) {

332                      BEQ conditional5

333 return5:                                                          ;unset($startStop)

334                      PUSH R3                                      ;$position = getButtonPressed(7)

335                      LOAD R3 7

336                      BRS _pressed

337                      PULL R3

338                      SUB SP 5

339                      PULL R3

340                      ADD SP 4

341                      CMP R3 0                                     ;if ($position == 0) {

342                      BEQ conditional6

343 return6:                                                          ;unset($position)

344                      PUSH R3                                      ;$colour = getButtonPressed(6)

345                      LOAD R3 6

346                      BRS _pressed

347                      PULL R3

348                      SUB SP 5

349                      PULL R3

350                      ADD SP 4

351                      CMP R3 1                                     ;if ($colour == 1) {

352                      BEQ conditional7

353 return7:                                                          ;unset($colour)

354                      BRA runningWait                              ;runningWait()

355

356                                                                   ;if ($startStop == 1) {

357 conditional5:        LOAD R4 0                                    ;$temp = 0

358                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

359                                                                   ;unset($temp)

360                      PUSH R5 ;reset timer                         ;setCountdown(BELT * 10)

361                      PUSH R4

362                      LOAD R5 -16

363                      LOAD R4 0

364                      SUB R4 [R5+13]

365                      STOR R4 [R5+13]                              ;set timer

366                      LOAD R4 BELT * 10

367                      STOR R4 [R5+13]

368                      PULL R4

369                      PULL R5

370                      LOAD R4 9                                    ;$state = 9

371                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

372                                                                   ;unset($state)

373                      BRA runningTimer                             ;runningTimer()

374

375                                                                   ;if ($position == 0) {

376 conditional6:        PUSH R5 ;reset timer                         ;setCountdown(COUNTDOWN)

377                      PUSH R4

378                      LOAD R5 -16

379                      LOAD R4 0

380                      SUB R4 [R5+13]

381                      STOR R4 [R5+13]                              ;set timer

382                      LOAD R4 COUNTDOWN

383                      STOR R4 [R5+13]

384                      PULL R4

385                      PULL R5

386                      LOAD R4 5                                    ;$state = 5

387                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

388                                                                   ;unset($state)

389                      BRA runningTimerReset                        ;runningTimerReset()

390

391                                                                   ;if ($colour == 1) {

392 conditional7:        LOAD R4 10                                   ;$temp = 10

393                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

394                      LOAD R4 0                                    ;$temp = 0

395                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

396                                                                   ;unset($temp)

397                      LOAD R4 6                                    ;$state = 6

398                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

399                                                                   ;unset($state)

400                      BRA motorUp                                  ;motorUp()

401

402 motorUp:             BRS timerManage                              ;timerManage()

403                      PUSH R3                                      ;$startStop = getButtonPressed(0)

404                      LOAD R3 0

405                      BRS _pressed

406                      PULL R3

407                      SUB SP 5

408                      PULL R3

409                      ADD SP 4

410                      CMP R3 1                                     ;if ($startStop == 1) {

411                      BEQ conditional8

412 return8:                                                          ;unset($startStop)

413                      PUSH R3                                      ;$push = getButtonPressed(5)

414                      LOAD R3 5

415                      BRS _pressed

416                      PULL R3

417                      SUB SP 5

418                      PULL R3

419                      ADD SP 4

420                      CMP R3 1                                     ;if ($push == 1) {

421                      BEQ conditional9

422 return9:                                                          ;unset($push)

423                      BRA motorUp                                  ;motorUp()

424

425                                                                   ;if ($startStop == 1) {

426 conditional8:        LOAD R4 0                                    ;$temp = 0

427                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

428                                                                   ;unset($temp)

429                      PUSH R5 ;reset timer                         ;setCountdown(BELT * 10)

430                      PUSH R4

431                      LOAD R5 -16

432                      LOAD R4 0

433                      SUB R4 [R5+13]

434                      STOR R4 [R5+13]                              ;set timer

435                      LOAD R4 BELT * 10

436                      STOR R4 [R5+13]

437                      PULL R4

438                      PULL R5

439                      LOAD R4 10                                   ;$state = 10

440                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

441                                                                   ;unset($state)

442                      BRA motorUpTimer                             ;motorUpTimer()

443

444                                                                   ;if ($push == 1) {

445 conditional9:        LOAD R4 0                                    ;$temp = 0

446                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

447                                                                   ;unset($temp)

448                      LOAD R4 7                                    ;$state = 7

449                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

450                                                                   ;unset($state)

451                      LOAD R2 0                                    ;$sleep = 0

452                      BRA whiteWait                                ;whiteWait()

453

454 whiteWait:           BRS timerManage                              ;timerManage()

455                      CMP R2 SORT                                  ;if ($sleep == SORT) {

456                      BEQ conditional10

457 return10:            PUSH R3                                      ;$startStop = getButtonPressed(0)

458                      LOAD R3 0

459                      BRS _pressed

460                      PULL R3

461                      SUB SP 5

462                      PULL R3

463                      ADD SP 4

464                      CMP R3 1                                     ;if ($startStop == 1) {

465                      BEQ conditional11

466 return11:                                                         ;unset($startStop)

467                      ADD R2 1                                     ;$sleep+=1

468                      BRA whiteWait                                ;whiteWait()

469

470                                                                   ;if ($sleep == SORT) {

471 conditional10:       LOAD R3 10                                   ;$temp = 10

472                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

473                                                                   ;unset($temp)

474                      PUSH R5 ;reset timer                         ;setCountdown(COUNTDOWN)

475                      PUSH R4

476                      LOAD R5 -16

477                      LOAD R4 0

478                      SUB R4 [R5+13]

479                      STOR R4 [R5+13]                              ;set timer

480                      LOAD R4 COUNTDOWN

481                      STOR R4 [R5+13]

482                      PULL R4

483                      PULL R5

484                      LOAD R3 8                                    ;$state = 8

485                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

486                                                                   ;unset($state)

487                      LOAD R2 0                                    ;$sleep = 0

488                      BRA motorDown                                ;motorDown()

489

490                                                                   ;if ($startStop == 1) {

491 conditional11:       LOAD R4 0                                    ;$temp = 0

492                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

493                                                                   ;unset($temp)

494                      PUSH R5 ;reset timer                         ;setCountdown(BELT * 10)

495                      PUSH R4

496                      LOAD R5 -16

497                      LOAD R4 0

498                      SUB R4 [R5+13]

499                      STOR R4 [R5+13]                              ;set timer

500                      LOAD R4 BELT * 10

501                      STOR R4 [R5+13]

502                      PULL R4

503                      PULL R5

504                      LOAD R4 11                                   ;$state = 11

505                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

506                                                                   ;unset($state)

507                      BRA whiteWaitTimer                           ;whiteWaitTimer()

508

509 whiteWaitTimer:      BRS timerManage                              ;timerManage()

510                      LOAD R3 15                                   ;$state = 15

511                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

512                                                                   ;unset($state)

513                      BRA whiteWaitStop                            ;whiteWaitStop()

514

515 whiteWaitStop:       BRS timerManage                              ;timerManage()

516                      CMP R2 SORT                                  ;if ($sleep == SORT) {

517                      BEQ conditional12

518 return12:            ADD R2 1                                     ;$sleep+=1

519                      BRA whiteWaitStop                            ;whiteWaitStop()

520

521                                                                   ;if ($sleep == SORT) {

522 conditional12:       LOAD R3 10                                   ;$temp = 10

523                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

524                      LOAD R3 0                                    ;$temp = 0

525                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

526                                                                   ;unset($temp)

527                      LOAD R3 12                                   ;$state = 12

528                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

529                                                                   ;unset($state)

530                      LOAD R2 0                                    ;$sleep = 0

531                      BRA motorDownStop                            ;motorDownStop()

532

533 motorDownStop:       BRS timerManage                              ;timerManage()

534                      CMP R2 TIMEMOTORDOWN                         ;if ($sleep == TIMEMOTORDOWN) {

535                      BEQ conditional13

536 return13:            ADD R2 1                                     ;$sleep+=1

537                      BRA motorDownStop                            ;motorDownStop()

538

539                                                                   ;if ($sleep == TIMEMOTORDOWN) {

540 conditional13:       LOAD R3 0                                    ;$temp = 0

541                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

542                                                                   ;unset($temp)

543                      LOAD R3 9                                    ;$state = 9

544                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

545                                                                   ;unset($state)

546                      LOAD R2 0                                    ;$sleep = 0

547                      BRA runningStop                              ;runningStop()

548

549 runningStop:         BRS timerManage                              ;timerManage()

550                      PUSH R3                                      ;$colour = getButtonPressed(6)

551                      LOAD R3 6

552                      BRS _pressed

553                      PULL R3

554                      SUB SP 5

555                      PULL R3

556                      ADD SP 4

557                      CMP R3 1                                     ;if ($colour == 1) {

558                      BEQ conditional14

559 return14:                                                         ;unset($colour)

560                      BRA runningStop                              ;runningStop()

561

562                                                                   ;if ($colour == 1) {

563 conditional14:       LOAD R4 10                                   ;$temp = 10

564                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

565                      LOAD R4 0                                    ;$temp = 0

566                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

567                                                                   ;unset($temp)

568                      LOAD R4 10                                   ;$state = 10

569                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

570                                                                   ;unset($state)

571                      BRA motorUpStop                              ;motorUpStop()

572

573 motorUpStop:         BRS timerManage                              ;timerManage()

574                      PUSH R3                                      ;$push = getButtonPressed(5)

575                      LOAD R3 5

576                      BRS _pressed

577                      PULL R3

578                      SUB SP 5

579                      PULL R3

580                      ADD SP 4

581                      CMP R3 1                                     ;if ($push == 1) {

582                      BEQ conditional15

583 return15:                                                         ;unset($push)

584                      BRA motorUpStop                              ;motorUpStop()

585

586                                                                   ;if ($push == 1) {

587 conditional15:       LOAD R4 0                                    ;$temp = 0

588                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

589                                                                   ;unset($temp)

590                      LOAD R4 11                                   ;$state = 11

591                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

592                                                                   ;unset($state)

593                      BRA whiteWaitStop                            ;whiteWaitStop()

594

595 motorDown:           BRS timerManage                              ;timerManage()

596                      PUSH R3                                      ;$colour = getButtonPressed(6)

597                      LOAD R3 6

598                      BRS _pressed

599                      PULL R3

600                      SUB SP 5

601                      PULL R3

602                      ADD SP 4

603                      CMP R3 1                                     ;if ($colour == 1) {

604                      BEQ conditional16

605 return16:                                                         ;unset($colour)

606                      CMP R2 TIMEMOTORDOWN                         ;if ($sleep == TIMEMOTORDOWN) {

607                      BEQ conditional17

608 return17:            PUSH R3                                      ;$startStop = getButtonPressed(0)

609                      LOAD R3 0

610                      BRS _pressed

611                      PULL R3

612                      SUB SP 5

613                      PULL R3

614                      ADD SP 4

615                      CMP R3 1                                     ;if ($startStop == 1) {

616                      BEQ conditional18

617 return18:                                                         ;unset($startStop)

618                      ADD R2 1                                     ;$sleep+=1

619                      BRA motorDown                                ;motorDown()

620

621                                                                   ;if ($colour == 1) {

622 conditional16:       LOAD R4 0                                    ;$temp=0

623                      STOR R4 [GB +outputs + HBRIDGE1]             ;storeData($temp,'outputs',HBRIDGE1)

624                      LOAD R4 10                                   ;$temp = 10

625                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

626                                                                   ;unset($temp)

627                      LOAD R4 6                                    ;$state = 6

628                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

629                      LOAD R2 0                                    ;$sleep=0

630                                                                   ;unset($state)

631                      BRA motorUp                                  ;motorUp()

632

633                                                                   ;if ($sleep == TIMEMOTORDOWN) {

634 conditional17:       LOAD R3 0                                    ;$temp = 0

635                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

636                      LOAD R3 7                                    ;$temp = 7

637                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

638                                                                   ;unset($temp)

639                      LOAD R3 4                                    ;$state = 4

640                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

641                      LOAD R2 0                                    ;$sleep = 0

642                                                                   ;unset($state)

643                      BRA runningWait                              ;runningWait()

644

645                                                                   ;if ($startStop == 1) {

646 conditional18:       LOAD R4 0                                    ;$temp = 0

647                      STOR R4 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

648                                                                   ;unset($temp)

649                      PUSH R5 ;reset timer                         ;setCountdown(BELT * 10)

650                      PUSH R4

651                      LOAD R5 -16

652                      LOAD R4 0

653                      SUB R4 [R5+13]

654                      STOR R4 [R5+13]                              ;set timer

655                      LOAD R4 BELT * 10

656                      STOR R4 [R5+13]

657                      PULL R4

658                      PULL R5

659                      LOAD R4 12                                   ;$state = 12

660                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

661                                                                   ;unset($state)

662                      BRA motorDownTimer                           ;motorDownTimer()

663

664 motorDownTimer:      BRS timerManage                              ;timerManage()

665                      LOAD R3 16                                   ;$state = 16

666                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

667                                                                   ;unset($state)

668                      BRA motorDownStop                            ;motorDownStop()

669

670 motorUpTimer:        BRS timerManage                              ;timerManage()

671                      LOAD R3 14                                   ;$state = 14

672                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

673                                                                   ;unset($state)

674                      BRA motorUpStop                              ;motorUpStop()

675

676 runningTimerReset:   BRS timerManage                              ;timerManage()

677                      LOAD R3 4                                    ;$state = 4

678                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

679                                                                   ;unset($state)

680                      BRA runningWait                              ;runningWait()

681

682 runningTimer:        BRS timerManage                              ;timerManage()

683                      LOAD R3 13                                   ;$state = 13

684                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

685                                                                   ;unset($state)

686                      BRA runningStop                              ;runningStop()

687

688                                                                   ;if ($location == 0) {

689 conditional19:       LOAD R3 0                                    ;$engines = 0

690                      BRA return19                                 ;}

691

692                                                                   ;if ($voltage > $counter) {

693 conditional20:       LOAD R4 R1                                   ;$voltage = $location

694                      PUSH R5                                      ;$voltage = pow(2, $voltage)

695                      LOAD R5 2

696                      BRS _pow

697                      LOAD R4 R5

698                      PULL R5

699                      ADD R3 R4                                    ;$engines += $voltage

700                      BRA return20                                 ;}

701

702                                                                   ;if ($location == 7) {

703 conditional21:       PUSH R5                                      ;sleep(1)

704                      LOAD R5 1

705                      BRS _timer

706                      PULL R5

707                      PUSH R5                                      ;display($engines, 'leds')

708                      LOAD R5 -16

709                      STOR R3 [R5+11]

710                      PULL R5

711                                                                   ;unset($voltage)

712                      PUSH R3                                      ;$abort = getButtonPressed(1)

713                      LOAD R3 1

714                      BRS _pressed

715                      PULL R3

716                      SUB SP 5

717                      PULL R4

718                      ADD SP 4

719                      CMP R4 1                                     ;if ($abort == 1) {

720                      BEQ conditional22

721 return22:                                                         ;unset($abort)

722                      CMP R0 6                                     ;if ($counter == 6) {

723                      BEQ conditional23

724 return23:            CMP R0 11                                    ;if ($counter == 11) {

725                      BEQ conditional24

726 return24:            LOAD R3 0                                    ;$engines = 0

727                      LOAD R1 0                                    ;$location = 0

728                      ADD R0 1                                     ;$counter+=1

729                      RTS                                          ;return

730                      BRA return21                                 ;}

731

732                                                                   ;if ($abort == 1) {

733 conditional22:       BRA abort                                    ;abort()

734                      BRA return22                                 ;}

735

736                                                                   ;if ($counter == 6) {

737 conditional23:       LOAD R4 [ GB + state + 0 ]                   ;$temp = getData('state', 0)

738                      MOD R4 10                                    ;mod(10, $temp)

739                      PUSH R5                                      ;display($temp, 'display', 1)

740                      PUSH R4

741                      LOAD R5 R4

742                      BRS _Hex7Seg

743                      LOAD R4 %0000001

744                      STOR R4 [R5+9]

745                      PULL R4

746                      PULL R5

747                                                                   ;unset($temp)

748                      BRA return23                                 ;}

749

750                                                                   ;if ($counter == 11) {

751 conditional24:       PUSH R2                                      ;pushStack($sleep)

752                      LOAD R4 [ GB + state + 0 ]                   ;$temp = getData('state', 0)

753                      LOAD R2 R4                                   ;$sleep = $temp

754                      MOD R2 10                                    ;mod(10, $sleep)

755                      SUB R4 R2                                    ;$temp -= $sleep

756                      DIV R4 10                                    ;$temp /= 10

757                      PUSH R5                                      ;display($temp, 'display', 2)

758                      PUSH R4

759                      LOAD R5 R4

760                      BRS _Hex7Seg

761                      LOAD R4 %0000010

762                      STOR R4 [R5+9]

763                      PULL R4

764                      PULL R5

765                      PULL R2                                      ;pullStack($sleep)

766                                                                   ;unset($temp)

767                      BRA return24                                 ;}

768

769 abort:                                                            ;unset($engines)

770                      PUSH R5 ;reset timer                         ;setCountdown(1000)

771                      PUSH R4

772                      LOAD R5 -16

773                      LOAD R4 0

774                      SUB R4 [R5+13]

775                      STOR R4 [R5+13]                              ;set timer

776                      LOAD R4 1000

777                      STOR R4 [R5+13]

778                      PULL R4

779                      PULL R5

780                      LOAD R3 [ GB + stackPointer + 0 ]            ;$temp = getData('stackPointer', 0)

781                      LOAD SP R3                                   ;setStackPointer($temp)

782                      LOAD R3 0                                    ;$temp = 0

783                      STOR R3 [GB +outputs + HBRIDGE1]             ;storeData($temp, 'outputs', HBRIDGE1)

784                      STOR R3 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

785                      STOR R3 [GB +outputs + LENSLAMPPOSITION]     ;storeData($temp, 'outputs', LENSLAMPPOSITION)

786                      STOR R3 [GB +outputs + LENSLAMPSORTER]       ;storeData($temp, 'outputs', LENSLAMPSORTER)

787                      STOR R3 [GB +outputs + LEDSTATEINDICATOR]    ;storeData($temp, 'outputs', LEDSTATEINDICATOR)

788                      STOR R3 [GB +outputs + DISPLAY]              ;storeData($temp, 'outputs', DISPLAY)

789                      STOR R3 [GB +outputs + CONVEYORBELT]         ;storeData($temp, 'outputs', CONVEYORBELT)

790                      STOR R3 [GB +outputs + FEEDERENGINE]         ;storeData($temp, 'outputs', FEEDERENGINE)

791                                                                   ;unset($temp)

792                      BRS timerManage                              ;timerManage()

793                      LOAD R3 17                                   ;$state = 17

794                      STOR R3 [GB +state + 0]                      ;storeData($state, 'state', 0)

795                      LOAD R3 7                                    ;$state = 7

796                      PUSH R5                                      ;display($state, 'leds2', 0)

797                      LOAD R5 -16

798                      STOR R3 [R5+10]

799                      PULL R5

800                                                                   ;unset($state)

801                      BRA aborted                                  ;aborted()

802

803 aborted:             PUSH R5 ;reset timer                         ;setCountdown(1000)

804                      PUSH R4

805                      LOAD R5 -16

806                      LOAD R4 0

807                      SUB R4 [R5+13]

808                      STOR R4 [R5+13]                              ;set timer

809                      LOAD R4 1000

810                      STOR R4 [R5+13]

811                      PULL R4

812                      PULL R5

813                      BRS timerManage                              ;timerManage()

814                      PUSH R3                                      ;$startStop = getButtonPressed(0)

815                      LOAD R3 0

816                      BRS _pressed

817                      PULL R3

818                      SUB SP 5

819                      PULL R3

820                      ADD SP 4

821                      CMP R3 1                                     ;if ($startStop == 1) {

822                      BEQ conditional25

823 return25:                                                         ;unset($startStop)

824                      BRA aborted                                  ;aborted()

825

826                                                                   ;if ($startStop == 1) {

827 conditional25:       LOAD R4 10                                   ;$temp = 10

828                      STOR R4 [GB +outputs + HBRIDGE0]             ;storeData($temp, 'outputs', HBRIDGE0)

829                                                                   ;unset($temp)

830                      LOAD R4 0                                    ;$state = 0

831                      STOR R4 [GB +state + 0]                      ;storeData($state, 'state', 0)

832                                                                   ;unset($state)

833                      BRA initial                                  ;initial()

834

835 timerManage:         CMP R1 0                                     ;if ($location == 0) {

836                      BEQ conditional19

837 return19:            MOD R0 12                                    ;mod(12, $counter)

838                      ADD R1 outputs                               ;$voltage = getData('outputs', $location)

839                      LOAD R4 [ GB + R1]

840                      SUB R1 outputs

841                      CMP R4 R0                                    ;if ($voltage > $counter) {

842                      BGT conditional20

843 return20:            CMP R1 7                                     ;if ($location == 7) {

844                      BEQ conditional21

845 return21:            ADD R1 1                                     ;$location+=1

846                      BRA timerManage                              ;branch('timerManage')

847

848                      @END